# DC Motor Working Principle | Back EMF & Its Significance | Torque Equation | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 2: Transformers and electrical machines  |  Presenter: Prof. Mitesh Astik

Topic: DC Motor: Principle, Back EMF and Torque Equation   |   Target length: 8:21 (1168 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:18 HOOK Visual: A motor starting from rest, with a current graph spiking and then dropping as speed rises. On-screen text: Why doesn't a running motor burn out? Animation: Animate a motor accelerating; the current graph spikes at start and falls as speed rises, while a glowing back-emf arrow opposes the supply. AI image/animation prompt: DC motor starting with a current spike graph falling as speed increases and a back emf arrow opposing the supply, flat style, 16:9 | When you start a ceiling fan, it slowly picks up speed. But electrically, at the moment of starting, a motor tries to draw a huge current. What protects the motor once it is running? An invisible voltage called the back emf. [41 words ≈ 18 sec] |
| 0:18 – 0:37 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 2: Transformers and electrical machines By the end you can: 1. Motor action 2. Voltage equation 3. Torque equation 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to explain motor action, derive the voltage equation with back emf, derive the torque equation, and solve a numerical. Let us begin. [46 words ≈ 20 sec] |
| 0:37 – 1:36 SCENE 1: PRINCIPLE AND CONSTRUCTION Visual: Current-carrying conductor in a magnetic field with force F = BIL; Fleming's left-hand rule; cross-section of a DC machine. On-screen text: A current-carrying conductor in a magnetic field experiences a force: F = B I L (N) Direction: Fleming's left-hand rule Parts: stator (yoke, poles, field winding), rotor (armature, commutator), brushes Commutator reverses the current in each coil → continuous rotation in one direction Animation: Draw a conductor between N and S poles with force arrow. Show Fleming's left-hand rule with the three fingers. Animate a coil rotating; the commutator flips the current direction every half turn. AI image/animation prompt: Current carrying conductor in a magnetic field with force arrow, left hand rule illustration, rotating armature coil with commutator and brushes, whiteboard style, 16:9 | A DC motor converts electrical energy into mechanical energy. When a conductor carrying current is placed in a magnetic field, it experiences a force equal to B I L. The direction is given by Fleming's left-hand rule: first finger for the field, second finger for the current, and thumb for the force. In a DC motor, the armature has many conductors. The stator provides the field through poles and a field winding. On the armature, the conductors on one side of the pole carry current in one direction, and those on the other side carry it in the opposite direction, so the forces make a couple, and the armature turns. The commutator and brushes reverse the current in a coil each time it passes the neutral axis, so that the torque always acts in the same direction. [137 words ≈ 59 sec] |
| 1:36 – 3:09 SCENE 2: BACK EMF AND THE VOLTAGE EQUATION Visual: Armature circuit: supply V, armature resistance Ra, back emf Eb opposing; current Ia. On-screen text: Rotating conductors cut flux → generator action → Eb (opposes V) Eb = (P Φ Z N)/(60 A) ∝ Φ N V = Eb + Ia·Ra → Ia = (V − Eb)/Ra Start: N = 0 → Eb = 0 → Ia(start) = V/Ra (very large) Running: Eb ≈ V → small Ia Eb is the regulator of the motor Animation: Draw the armature equivalent circuit. Animate the speed rising: Eb arrow grows, and Ia reading falls. Show the voltage equation with colour-coded terms. Highlight start-up current as a huge value. AI image/animation prompt: Armature equivalent circuit with supply V armature resistance Ra and back emf Eb, current reading falling as speed rises, voltage equation colour coded, 16:9 | Here is the beautiful feedback. As the armature conductors rotate in the magnetic field, they cut the flux, and by Faraday's law, an emf is induced in them. By Lenz's law, it opposes the cause, which is the applied voltage. So it is called the back emf, E b. It is proportional to the flux and the speed. The voltage equation of the motor is V equal to E b plus I a R a. So the armature current is V minus E b, divided by R a. At the moment of starting, the speed is zero, so E b is zero, and the current is V by R a, which can be many times the rated current, because R a is small. As the motor speeds up, E b rises, and the current falls. At the normal speed, E b is a little less than V, and the current matches the load. So the back emf automatically adjusts the current to the load. If the load increases, the speed drops slightly, E b drops, more current flows, and more torque is produced. This is the significance of the back emf. The brushes and commutator are the most maintenance-intensive part of a DC motor. Modern brushless motors replace them with electronic switching, but the principle is the same. [218 words ≈ 93 sec] |
| 3:09 – 4:03 SCENE 3: TORQUE EQUATION AND POWER Visual: Torque from the conductor force; mechanical power Eb·Ia = ω·T. On-screen text: T = k·Φ·Ia (torque ∝ flux × armature current) T = (P Z Φ Ia)/(2π A) N·m Mechanical power developed Pm = Eb·Ia = ωT = 2πNT/60 T ∝ Ia for constant Φ Starter: external resistance in series with armature at start Animation: Derive T by combining Eb·Ia = ωT; show the substitution step by step. Draw a starter (3-point) as a variable resistance in series with the armature, reduced as the speed rises. AI image/animation prompt: Torque equation derivation from mechanical power balance, and a 3 point starter with a variable resistor in series with the armature, whiteboard style, 16:9 | The torque equation follows from power. The electrical power converted to mechanical form is E b into I a. This must equal the mechanical power, omega times torque, where omega is two pi N by 60. Substituting E b proportional to flux and speed, the speed cancels, and we find T equals a constant times flux times armature current. So for a constant field, the torque is proportional to armature current. Because the starting current is so high, a starter is used. It is a variable resistance inserted in series with the armature at starting, and gradually cut out as the motor gains speed and its back emf builds up. This limits the starting current to about 1.5 to 2 times the full-load value. [124 words ≈ 53 sec] |
| 4:03 – 5:30 SOLVED NUMERICAL Visual: 220 V DC shunt motor: Ra = 0.5 Ω, Ia = 20 A, N = 1000 rpm. On-screen text: Eb = V − Ia·Ra = 220 − 20 × 0.5 = 210 V Pm = Eb·Ia = 210 × 20 = 4200 W ω = 2π × 1000/60 = 104.72 rad/s T = Pm/ω = 4200/104.72 = 40.1 N·m Starting current (no starter) = V/Ra = 220/0.5 = 440 A = 22 × rated current Starter to limit start current to 40 A: total R = 220/40 = 5.5 Ω → external R = 5.5 − 0.5 = 5.0 Ω Animation: Draw the armature circuit. Compute Eb, Pm, T in sequence with a motor shaft animation. Show the 440 A starting current as a tall red bar vs 20 A rated; then add the starter resistance and the start current bar shrinking to 40 A. AI image/animation prompt: DC motor armature circuit with back emf calculations, torque and power values, and a bar chart of starting current without and with a starter resistance, whiteboard style, 16:9 | A 220 volt DC shunt motor has an armature resistance of 0.5 ohm. When running at 1000 rpm, it takes an armature current of 20 amperes. Find the back emf, the mechanical power developed, the torque, the starting current without a starter, and the starter resistance to limit the starting current to 40 amperes. Step one: the back emf. E b equals V minus I a R a, equals 220 minus 20 into 0.5, equals 210 volts. Step two: the mechanical power. P m equals E b into I a, equals 210 into 20, equals 4,200 watts. Step three: the angular speed is 2 pi into 1000 by 60, equals 104.72 radians per second. The torque equals 4,200 by 104.72, equals 40.1 newton-metres. Step four: the starting current. At standstill, E b is zero, so I a equals 220 by 0.5, equals 440 amperes. That is 22 times the running current! The brush gear and the windings would be damaged. Step five: the starter. To limit the current to 40 amperes, the total resistance must be 220 by 40, equals 5.5 ohm. The armature already has 0.5, so the external resistance is 5.0 ohm. As the motor speeds up, this resistance is gradually cut out. [205 words ≈ 88 sec] |
| 5:30 – 6:58 COMMON MISTAKES AND EXAM TIPS Visual: DC motor mistakes. On-screen text: 1. Back emf opposes the supply 2. Eb is less than V in a motor (more in a generator) 3. Ia = (V − Eb)/Ra 4. Use rad/s for torque calculation Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: in a motor, the back emf opposes the supply voltage, and is always a little less than the supply. In a generator, it is the opposite. Two: use I a equal to V minus E b, divided by R a. Do not use V divided by R a, except at the moment of starting. Three: convert rpm to radians per second when calculating torque. Four: if the field winding fails open in a shunt motor, the flux falls almost to zero, and the speed can rise dangerously. So a shunt motor must never be started with its field open. That fourth point is also a common viva question. Remember that speed is inversely proportional to flux, so very low flux means a very high speed. A few more facts. There are three common types of DC motor, according to how the field is connected: shunt, series and compound. The series motor has a very high starting torque, so it is used for traction and cranes. The shunt motor has nearly constant speed, used for lathes and fans. To reverse the direction of rotation, reverse either the armature current or the field current, but not both, because reversing both leaves the direction unchanged. [205 words ≈ 88 sec] |
| 6:58 – 7:33 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. Motor action: F = BIL, Fleming's left-hand rule 2. Commutator keeps the torque in one direction 3. V = Eb + IaRa ; Eb ∝ ΦN 4. Back emf regulates the armature current 5. T = kΦIa ; Pm = Eb·Ia = ωT Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: motor action comes from the force B I L on a current carrying conductor Two: the commutator keeps the torque unidirectional Three: the voltage equation is V equals E b plus I a R a, and E b is proportional to flux and speed Four: the back emf automatically regulates the armature current to match the load Five: torque is proportional to flux and armature current, and mechanical power is E b I a [81 words ≈ 35 sec] |
| 7:33 – 8:21 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: 230 V DC motor, Ra = 0.2 Ω, Ia = 50 A. Find Eb and the mechanical power developed. Practice 2: 230 V, Ra = 0.4 Ω. Find the external starter resistance to limit the start current to 50 A. Next: DC Shunt Motor Characteristics and Speed Control Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A DC shunt motor with a rheostat in the armature and another in the field circuit, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a 230 volt DC motor has an armature resistance of 0.2 ohm and takes 50 amperes. Find the back emf and the mechanical power developed. Question two: 230 volts, armature resistance 0.4 ohm. Find the external starter resistance to limit the starting current to 50 amperes. Answers are in the pinned comment. In the next video, we study the characteristics of the DC shunt motor, and two ways to control its speed: armature control and field control. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [111 words ≈ 48 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Eb = 230 − 10 = 220 V ; Pm = 220 × 50 = 11 kW.

Practice 2 answer: Rtotal = 230/50 = 4.6 Ω ; Rext = 4.6 − 0.4 = 4.2 Ω.

# YouTube package

### Video title

DC Motor Working Principle | Back EMF & Its Significance | Torque Equation | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the working principle of a DC motor, back emf and its significance, the voltage equation, the torque equation, and why a starter is needed, with a solved numerical.

Chapters:

0:00 Hook

0:18 Learning objectives

0:37 Principle And Construction

1:36 Back Emf And The Voltage Equation

3:09 Torque Equation And Power

4:03 Solved Numerical

5:30 Common Mistakes And Exam Tips

6:58 Recap

7:33 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

DC motor, back emf, torque equation of DC motor, DC motor working principle, starter for DC motor, armature current, DC shunt motor, basic electrical engineering, BEEE, Prof Mitesh Astik

### Thumbnail text idea

DC MOTOR  |  Eb = V − IaRa  |  Why Starter Is Needed

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation <-- THIS VIDEO Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |