# DC Shunt Motor Speed Control | Armature Control & Field Control | Characteristics | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 2: Transformers and electrical machines  |  Presenter: Prof. Mitesh Astik

Topic: DC Shunt Motor Characteristics and Speed Control   |   Target length: 8:12 (1149 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:18 HOOK Visual: A lift going up and down with a speed control knob and a motor. On-screen text: A knob controls the speed Animation: Animate a lift and a rolling mill; a knob adjusts a rheostat symbol; the motor's speed gauge rises and falls. AI image/animation prompt: Lift and rolling mill driven by DC motors with a control knob and rheostat and a speed gauge, flat illustration, 16:9 | Electric trains, lifts and rolling mills all need smooth control of motor speed. DC motors have long been the first choice for this. How do we change the speed of a DC motor with just a knob? Let us find out. [41 words ≈ 18 sec] |
| 0:18 – 0:37 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 2: Transformers and electrical machines By the end you can: 1. Speed equation 2. Characteristics 3. Speed control methods 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to state the speed equation, draw the shunt-motor characteristics, apply armature and field control, and solve a numerical. Let us begin. [45 words ≈ 19 sec] |
| 0:37 – 1:25 SCENE 1: THE SPEED EQUATION Visual: Derivation of N ∝ Eb/Φ from Eb = kΦN and V = Eb + IaRa. On-screen text: Eb = V − Ia·Ra Eb = k·Φ·N → N = (V − Ia·Ra)/(k·Φ) N ∝ Eb/Φ So speed can be changed by: V (armature voltage), Ra (armature resistance), or Φ (field flux) For two states: N2/N1 = (Eb2/Eb1) × (Φ1/Φ2) Animation: Derive the equation line by line; then show three knobs on the circuit labelled V, Ra, and Φ, each with an arrow to speed N. AI image/animation prompt: Speed equation derivation with three knobs for armature voltage armature resistance and field flux each affecting speed, whiteboard style, 16:9 | To control speed, we start with the speed equation. From the voltage equation, E b equals V minus I a R a. And E b is proportional to flux and speed. So speed equals V minus I a R a, divided by k times flux. In other words, the speed is proportional to the back emf and inversely proportional to the flux. This tells us three ways to change the speed: change the armature voltage, change the resistance in the armature circuit, or change the field flux. The ratio form is useful in numericals: N 2 by N 1 equals E b 2 by E b 1, times flux 1 by flux 2. [113 words ≈ 48 sec] |
| 1:25 – 2:12 SCENE 2: CHARACTERISTICS OF THE SHUNT MOTOR Visual: Three graphs: speed vs armature current, torque vs armature current, speed vs torque. On-screen text: N–Ia: nearly constant (drops 3–5 % from no-load to full-load) — a constant speed motor T–Ia: straight line (T ∝ Ia, since Φ constant) N–T: slightly drooping Applications: lathes, fans, pumps, machine tools Animation: Draw the three graphs one by one, each with a short annotation. Show a lathe and a pump icon beside the speed-torque curve. AI image/animation prompt: Three characteristic graphs for DC shunt motor: speed versus armature current nearly flat, torque versus armature current straight, speed versus torque slightly drooping, 16:9 | In a shunt motor, the field winding is connected directly across the supply, so the flux is almost constant. Therefore, the speed drops only slightly, by 3 to 5 percent, as the load increases from no-load to full-load, because only the I a R a drop changes. So we call it a constant-speed motor. The torque is proportional to the armature current, so the torque-versus-current graph is a straight line. That makes the shunt motor ideal for loads that need a nearly constant speed: lathes, fans, pumps and machine tools. And because the field is separate from the armature, the speed can be adjusted over a wide range. [108 words ≈ 46 sec] |
| 2:12 – 3:27 SCENE 3: ARMATURE AND FIELD CONTROL Visual: Two circuits: rheostat in series with the armature; rheostat in series with the field. On-screen text: Armature (resistance) control: R in series with armature → speed BELOW rated Eb ↓ → N ↓ ; wasteful (I²R loss), poor regulation Field control: rheostat in the field → Φ ↓ → speed ABOVE rated Efficient (small field current), but limited range (commutation, stability) Ward-Leonard method: variable armature voltage (smooth, both directions) Animation: Draw the two circuits side by side. Animate the rheostat knob in each. In the armature-control circuit show heat leaving the series resistor. In the field circuit show the flux lines weakening and the speed gauge rising above the rated mark. AI image/animation prompt: Two DC shunt motor circuits, one with a rheostat in series with the armature, another with a rheostat in the field winding, speed gauges below and above the rated speed, 16:9 | Armature resistance control: add an external resistance in series with the armature. At the same load current, the voltage drop increases, so E b falls, and so does the speed. This gives speeds below the rated speed. It is simple, but the series resistance wastes power as heat, and the speed regulation is poor. Field control: add a rheostat in the field circuit. Reducing the field current reduces the flux, and the speed rises, since the speed is inversely proportional to the flux. This gives speeds above the rated speed. It is efficient, because the field current is small, but there is a limit. Too weak a field causes poor commutation and mechanical stress. A third method, used in industry, varies the voltage applied to the armature, as in the Ward-Leonard system, or with modern power electronics. It gives smooth control with high efficiency. Quick comparison to remember: armature control is like pressing the brake to slow a car, wasteful but simple. Field control is like lightening the load on the engine so the car speeds up. [177 words ≈ 76 sec] |
| 3:27 – 5:19 SOLVED NUMERICAL Visual: 220 V shunt motor: Ra = 0.4 Ω, Ia = 25 A, N = 1000 rpm. (a) Armature control to reach 800 rpm at the same torque. (b) Field weakened by 10 % at constant load torque. On-screen text: Eb1 = 220 − 25 × 0.4 = 210 V (a) Same torque, same Φ → Ia = 25 A. N2/N1 = Eb2/Eb1 → Eb2 = 210 × 0.8 = 168 V 220 − 25(0.4 + R) = 168 → 0.4 + R = 2.08 → R = 1.68 Ω (b) Φ2 = 0.9 Φ1 ; constant torque → Φ·Ia constant → Ia2 = 25/0.9 = 27.78 A Eb2 = 220 − 27.78 × 0.4 = 208.89 V N2 = 1000 × (208.89/210) × (1/0.9) = 1105 rpm Animation: Show the circuit with two modes. For (a), the series resistor appears and the speed gauge falls to 800. For (b), the field rheostat turns and the speed rises to 1105. Calculation steps appear beside each. AI image/animation prompt: DC shunt motor circuit with calculations for series resistance giving 800 rpm and field weakening giving 1105 rpm, speed gauges shown, whiteboard style, 16:9 | A 220 volt shunt motor has an armature resistance of 0.4 ohm. At a load that takes 25 amperes, it runs at 1000 rpm. Part A: what resistance in series with the armature gives 800 rpm at the same torque? Part B: what is the speed if the field flux is reduced by 10 percent, with the same load torque? First the back emf at 1000 rpm. E b 1 equals 220 minus 25 into 0.4, equals 210 volts. Part A. At the same torque and the same flux, the armature current is still 25 amperes. The speed ratio equals the back emf ratio. So E b 2 equals 210 into 800 by 1000, equals 168 volts. The voltage equation: 220 minus 25 times 0.4 plus R, equals 168. So 0.4 plus R equals 52 by 25, equals 2.08, and R equals 1.68 ohm. Part B. Now the flux falls to 0.9 of the original. The torque is proportional to flux times armature current, and the load torque is constant. So the current rises to 25 by 0.9, equals 27.78 amperes. The new back emf is 220 minus 27.78 into 0.4, equals 208.89 volts. The speed ratio equals the back emf ratio times the inverse flux ratio. N 2 equals 1000 into 208.89 by 210, into 1 by 0.9. That is 1000 into 0.9947 into 1.111, equals 1105 rpm. A 10 percent weaker field gives a 10.5 percent higher speed. Notice that the current also rose by 11 percent, so do not over-weaken the field on a heavily loaded motor. [260 words ≈ 111 sec] |
| 5:19 – 6:56 COMMON MISTAKES AND EXAM TIPS Visual: Speed control mistakes. On-screen text: 1. Armature R → speed BELOW rated; field control → ABOVE rated 2. Constant torque means Φ·Ia stays constant 3. Use Eb, not V, in the speed ratio 4. Never open the shunt field Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: armature resistance control reduces speed below the rated value, and field control raises it above. Two: when the problem says constant torque and the flux changes, the armature current changes in the opposite direction, so that flux times current stays constant. Three: in the speed ratio, use the back emfs, not the supply voltages. Four: never open the shunt field circuit of a running motor, or start it with the field open. The flux falls to the residual value, and the speed can rise to a dangerous level. That is why field rheostats are designed with a minimum limit. A good viva answer for why the shunt motor is called a constant-speed motor: because the flux is nearly constant, and the I a R a drop is small compared to the supply voltage. One more definition: speed regulation. It is the change in speed from no load to full load, expressed as a percentage of the full-load speed. For example, if the no-load speed is 1040 rpm and the full-load speed is 1000 rpm, the speed regulation is 40 by 1000, which is 4 percent. A small value means the speed is nearly constant. Contrast this with a series motor, whose speed falls sharply as the load rises, and which can race dangerously at no load, so it must never be started without load. [227 words ≈ 97 sec] |
| 6:56 – 7:21 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. N ∝ Eb/Φ ; N = (V − IaRa)/(kΦ) 2. Shunt motor: nearly constant speed 3. Armature resistance control: below rated speed, wasteful 4. Field control: above rated speed, efficient 5. At constant torque, weaker flux → higher Ia Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: speed is proportional to back emf and inversely proportional to flux Two: the shunt motor runs at nearly constant speed Three: armature resistance control gives speeds below rated, with wasted power Four: field control gives speeds above rated, and is efficient Five: at constant torque a weaker flux raises the armature current [59 words ≈ 25 sec] |
| 7:21 – 8:12 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: 220 V, Ra = 0.5 Ω, Ia = 20 A, N = 900 rpm. A 2 Ω resistor is added in series with the armature (same Ia). Find the new speed. Practice 2: The motor in the worked example has its flux reduced by 20 % at constant torque. Find the new speed. Next: Three-Phase Induction Motor Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A three-phase stator with a rotating magnetic field and a squirrel cage rotor, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a 220 volt motor has Ra equal to 0.5 ohm, draws 20 amperes, and runs at 900 rpm. A 2 ohm resistor is added in the armature circuit, with the same current. Find the new speed. Question two: the motor in the worked example has its flux reduced by 20 percent at constant torque. Find the new speed. Answers are in the pinned comment. In the next video, we study the workhorse of industry, the three-phase induction motor: rotating magnetic field, principle, construction and types. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [119 words ≈ 51 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Eb1 = 210 V ; Eb2 = 220 − 20 × 2.5 = 170 V ; N2 = 900 × 170/210 = 728.6 rpm.

Practice 2 answer: Ia2 = 25/0.8 = 31.25 A ; Eb2 = 220 − 12.5 = 207.5 V ; N2 = 1000 × (207.5/210)/0.8 = 1235 rpm.

# YouTube package

### Video title

DC Shunt Motor Speed Control | Armature Control & Field Control | Characteristics | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the characteristics of the DC shunt motor and two methods of speed control, armature resistance control and field flux control, with a solved numerical.

Chapters:

0:00 Hook

0:18 Learning objectives

0:37 The Speed Equation

1:25 Characteristics Of The Shunt Motor

2:12 Armature And Field Control

3:27 Solved Numerical

5:19 Common Mistakes And Exam Tips

6:56 Recap

7:21 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

DC shunt motor, speed control of DC motor, armature control, field control, motor characteristics, torque speed characteristics, N proportional to Eb by flux, basic electrical engineering, BEEE, Prof Mitesh Astik

### Thumbnail text idea

SPEED CONTROL  |  Armature Resistance  |  Field Weakening  |  1105 rpm

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control <-- THIS VIDEO Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |