# Three Phase Induction Motor | Rotating Magnetic Field, Slip, Squirrel Cage vs Wound Rotor | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 2: Transformers and electrical machines  |  Presenter: Prof. Mitesh Astik

Topic: Three-Phase Induction Motor   |   Target length: 8:18 (1163 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:20 HOOK Visual: A factory floor with pumps, fans and conveyors, all driven by induction motors. On-screen text: No wires to the rotor. Yet it spins. Animation: Animate a factory scene with multiple motors; zoom into one motor to show a rotating field around a rotor with no connection to the supply. AI image/animation prompt: Factory floor with several three phase induction motors on pumps and conveyors, zoom into a motor showing rotating magnetic field around a rotor, flat illustration, 16:9 | Walk through any factory: the pumps, the fans, the conveyor belts, the lifts. Most of them are run by one type of motor, the three-phase induction motor. It has no brushes, no commutator, and no electrical connection to the rotor at all. How does it turn? [46 words ≈ 20 sec] |
| 0:20 – 0:40 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 2: Transformers and electrical machines By the end you can: 1. Rotating magnetic field 2. Working principle 3. Speed and slip 4. Rotor types Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to explain the rotating magnetic field, describe the working principle, define synchronous speed and slip, and compare squirrel-cage and wound-rotor types. Let us begin. [48 words ≈ 21 sec] |
| 0:40 – 1:54 SCENE 1: THE ROTATING MAGNETIC FIELD Visual: Three stator windings 120° apart carrying three-phase currents; the resultant field rotating. On-screen text: Three windings 120° apart, carrying currents 120° apart in time Resultant flux: constant magnitude = 1.5 Φm, rotating at synchronous speed Ns = 120 f / P (rpm) Reverse any two phases → rotation reverses Animation: Draw the three-phase currents as three sine waves. At six time instants, draw the resultant flux vector, rotating step by step. Show the Ns formula and an example for P = 4, f = 50 Hz. AI image/animation prompt: Three phase stator windings 120 degrees apart, three sine waves of currents and the resultant flux vector rotating at six time instants, whiteboard style, 16:9 | The secret of the induction motor is the rotating magnetic field. The stator has three windings, placed 120 degrees apart in space. When a three-phase supply is connected, the currents in them are also 120 degrees apart in time. Take any instant, and add the three fluxes as vectors: the resultant has a constant magnitude of 1.5 times the maximum flux of one phase. At the next instant, the same resultant, but turned by an angle. The field rotates, with constant magnitude. Its speed is the synchronous speed N s equal to 120 f divided by P, in rpm, where f is the supply frequency and P the number of poles. For a 4-pole machine on a 50 hertz supply, N s is 1500 rpm. If you interchange any two supply leads, the direction of rotation of the field, and of the motor, reverses. The rotating field has a constant magnitude, not a pulsating one. That is the beauty of a three-phase supply, and why three-phase motors start by themselves, unlike single-phase ones. [173 words ≈ 74 sec] |
| 1:54 – 3:08 SCENE 2: WORKING PRINCIPLE AND SLIP Visual: Rotating field cutting the rotor conductors; induced emf; current; force; rotor follows but lags. On-screen text: Rotating field cuts rotor conductors → emf (Faraday) Closed rotor circuit → rotor current → force (Lorentz) → torque Rotor follows the field (Lenz) but N < Ns — otherwise no relative motion → no emf Slip s = (Ns − N)/Ns Rotor frequency fr = s·f Typical full-load slip 2–6 % Animation: Animate the rotating field around a rotor; as the rotor accelerates, show relative speed falling and the induced currents shrinking. At full speed, a small relative gap remains. Write the slip formula with the numbers. AI image/animation prompt: Rotating magnetic field cutting rotor conductors with induced current and force arrows, rotor speed slightly below synchronous speed, slip formula, whiteboard style, 16:9 | How does the rotor turn, when it is not connected to any supply? At the start, the rotor is at rest, and the rotating field cuts its conductors, inducing an emf by Faraday's law. The rotor circuit is closed, so a current flows. This current, in the stator field, experiences a force, and creates a torque. By Lenz's law, the rotor follows the field to reduce the relative speed. But it can never reach the synchronous speed. If it did, there would be no relative motion, no emf, no current, and no torque. So the rotor runs at a speed N slightly less than N s. The difference, expressed as a fraction of N s, is the slip: s equals N s minus N, divided by N s. At full load, the slip is typically 2 to 6 percent. The rotor frequency is the slip times the supply frequency. At standstill, s is 1 and the rotor frequency equals the supply frequency. At full speed, it is only a few hertz. [171 words ≈ 73 sec] |
| 3:08 – 4:02 SCENE 3: CONSTRUCTION AND TYPES Visual: Cutaway of stator (laminated core with slots, three-phase winding) and two rotors: squirrel cage and wound rotor with slip rings. On-screen text: Stator: laminated steel core + 3-phase winding in slots Squirrel-cage rotor: bars short-circuited by end rings; simple, rugged, cheap; low starting torque; most common Wound rotor (slip-ring): 3-phase winding to slip rings and brushes; external resistance → high starting torque, low starting current; costlier, more maintenance Animation: Show the cutaway. Animate the aluminium bars and end rings forming the 'cage' separately. Then the wound rotor with slip rings and an external rheostat. Table of comparison appears. AI image/animation prompt: Cutaway of induction motor stator and two rotors: squirrel cage with bars and end rings, and wound rotor with slip rings and external resistance, comparison table, 16:9 | The stator is the same in both types: a laminated core with slots, carrying a three-phase winding. The rotor comes in two types. The squirrel-cage rotor has copper or aluminium bars in the slots, short-circuited at both ends by end rings, like a cage. It is simple, rugged, cheap, and needs almost no maintenance. It has low starting torque, but it is used in the great majority of applications. The wound rotor, or slip-ring rotor, has a three-phase winding, connected through slip rings and brushes to external resistances. By adding resistance at starting, we get high starting torque with low starting current, then cut the resistance out. It is more expensive, and needs more maintenance, so it is used for heavy-starting loads like cranes and mills. [126 words ≈ 54 sec] |
| 4:02 – 5:30 SOLVED NUMERICAL Visual: (a) 4-pole, 50 Hz induction motor running at 1440 rpm. (b) Stator input 10 kW, stator losses 0.5 kW, slip 4 %. On-screen text: (a) Ns = 120 × 50/4 = 1500 rpm s = (1500 − 1440)/1500 = 0.04 = 4 % Rotor frequency fr = s f = 0.04 × 50 = 2 Hz Same motor with 6 poles: Ns = 1000 rpm (b) Air-gap power Pg = 10 − 0.5 = 9.5 kW Rotor copper loss = s × Pg = 0.04 × 9500 = 380 W Mechanical power developed = (1 − s) Pg = 0.96 × 9500 = 9120 W Animation: Show the motor with a speed gauge reading 1440 under a synchronous marker of 1500; the gap highlights the slip. Compute each step. For (b), show a power-flow diagram: input → stator losses → air-gap → rotor copper loss → mechanical. AI image/animation prompt: Induction motor speed gauge showing 1440 rpm against synchronous speed 1500, slip calculation, and power flow diagram with stator loss rotor copper loss and mechanical output, 16:9 | A 4-pole, 50 hertz induction motor runs at 1440 rpm. Find the synchronous speed, slip, and rotor frequency. And if the stator input is 10 kilowatts, with stator losses of 0.5 kilowatt, find the rotor copper loss and the mechanical power developed, at this slip. Step one: N s equals 120 into 50 divided by 4, equals 1500 rpm. Step two: the slip equals 1500 minus 1440, divided by 1500, equals 60 by 1500, equals 0.04, or 4 percent. Step three: the rotor frequency equals slip times supply frequency, equals 0.04 into 50, equals 2 hertz. The rotor current changes direction only 2 times a second. Step four: the power flow. Of the 10 kilowatt input, 0.5 kilowatt is lost in the stator, so 9.5 kilowatt crosses the air gap to the rotor. The rotor copper loss equals slip times the air-gap power, equals 0.04 into 9,500, equals 380 watts. And the mechanical power developed equals 1 minus slip, times the air-gap power, equals 0.96 into 9,500, equals 9,120 watts. Notice that the rotor efficiency equals 1 minus s, which is why a low slip means an efficient motor. If the same motor had 6 poles, the synchronous speed would be 1000 rpm. More poles, lower speed. [207 words ≈ 89 sec] |
| 5:30 – 7:01 COMMON MISTAKES AND EXAM TIPS Visual: Induction motor mistakes. On-screen text: 1. Slip is a fraction, not a speed 2. Rotor speed is below Ns, always 3. Use poles P, not pole pairs, in 120f/P 4. Reverse any TWO phases to reverse direction Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: slip is a fraction of the synchronous speed, not the speed difference itself. Two: the rotor of an induction motor always runs below the synchronous speed in motoring. If a problem gives a rotor speed above synchronous, it is not motor operation. Three: use the number of poles, P, in 120 f by P, not the pairs. Four: to reverse the direction of the motor, interchange any two of the three supply lines. Interchanging all three does nothing. Also remember a safety point: a squirrel-cage motor draws five to seven times its rated current at starting, which is why starters like the star-delta are used for large motors. Starting is important, because the starting current of a squirrel-cage motor is five to seven times the rated current. For small motors, direct-on-line starting is acceptable. For larger motors, the star-delta starter connects the stator windings in star at starting, and then in delta at running speed. In star, the phase voltage is reduced by root 3, so the line current at starting is one third of that with direct delta connection. The starting torque is also reduced to one third, so the load must be light at starting. Autotransformer starters and rotor resistance starters, for wound-rotor motors, are other options. [212 words ≈ 91 sec] |
| 7:01 – 7:36 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. RMF: 3 windings 120° apart + 3-phase supply → constant-magnitude rotating field 2. Ns = 120f/P 3. Slip s = (Ns − N)/Ns ; fr = s f 4. Squirrel cage: simple; wound rotor: high starting torque 5. Rotor copper loss = s × Pg ; Pm = (1 − s)Pg Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: a three phase winding on a three phase supply produces a rotating field of constant magnitude Two: synchronous speed is 120 f by P Three: slip is the fractional speed difference, and rotor frequency is s times f Four: the squirrel cage is simple and rugged, while the wound rotor gives high starting torque Five: rotor copper loss is s times the air-gap power, and mechanical power is 1 minus s times the air-gap power [82 words ≈ 35 sec] |
| 7:36 – 8:18 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: A 6-pole, 50 Hz induction motor has 5 % slip. Find Ns and the rotor speed. Practice 2: An 8-pole, 50 Hz motor runs at 720 rpm. Find the slip and the rotor frequency. Next: Semiconductor Diode and Its Equivalent Circuits Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A PN junction with depletion region and a diode symbol, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a 6-pole, 50 hertz induction motor has 5 percent slip. Find the synchronous speed and the rotor speed. Question two: an 8-pole, 50 hertz motor runs at 720 rpm. Find the slip and the rotor frequency. Answers are in the pinned comment. In the next video, we begin Unit 3, diodes and transistors, with the PN junction diode, its characteristics and its equivalent circuits. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [98 words ≈ 42 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Ns = 1000 rpm ; N = 1000 × 0.95 = 950 rpm.

Practice 2 answer: Ns = 750 rpm ; s = 30/750 = 0.04 ; fr = 2 Hz.

# YouTube package

### Video title

Three Phase Induction Motor | Rotating Magnetic Field, Slip, Squirrel Cage vs Wound Rotor | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the three-phase induction motor: how a rotating magnetic field is produced, the working principle, synchronous speed and slip, squirrel-cage and wound-rotor constructions, with a worked numerical.

Chapters:

0:00 Hook

0:20 Learning objectives

0:40 The Rotating Magnetic Field

1:54 Working Principle And Slip

3:08 Construction And Types

4:02 Solved Numerical

5:30 Common Mistakes And Exam Tips

7:01 Recap

7:36 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

three phase induction motor, rotating magnetic field, slip, synchronous speed, squirrel cage rotor, wound rotor, induction motor working principle, basic electrical engineering, BEEE, Prof Mitesh Astik

### Thumbnail text idea

INDUCTION MOTOR  |  Ns = 120f/P  |  Slip = 4%  |  Rotor f = 2 Hz

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor <-- THIS VIDEO |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |