# PN Junction Diode | V-I Characteristics & Equivalent Circuits (Ideal, Simplified, Piecewise Linear) | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 3: Diodes, transistors and applications  |  Presenter: Prof. Mitesh Astik

Topic: Semiconductor Diode and Equivalent Circuits   |   Target length: 8:13 (1151 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:17 HOOK Visual: A phone charger being opened to show a diode, with a one-way valve analogy beside it. On-screen text: A one-way gate for current Animation: Animate a water pipe with a one-way valve beside a diode symbol; current flows in one direction and is blocked in the other. AI image/animation prompt: One way valve in a water pipe next to a diode symbol with current flow and block, flat illustration, 16:9 | Every phone charger, every LED bulb driver, every radio has the same tiny component at the heart of it: a one-way gate for electric current, the diode. Today we learn how it works and how to analyse circuits with it. [40 words ≈ 17 sec] |
| 0:17 – 0:36 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 3: Diodes, transistors and applications By the end you can: 1. PN junction 2. V-I characteristic 3. Equivalent circuits 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to explain the PN junction, draw the V-I characteristic, use diode equivalent circuits, and solve a numerical. Let us begin. [44 words ≈ 19 sec] |
| 0:36 – 1:35 SCENE 1: THE PN JUNCTION Visual: P and N regions meeting; depletion region and barrier potential; forward and reverse bias. On-screen text: P-type (holes) + N-type (electrons) → junction Depletion region, barrier potential: Si ≈ 0.7 V , Ge ≈ 0.3 V Forward bias (P to +): barrier ↓ → current flows Reverse bias (P to −): barrier ↑ → only tiny leakage current (µA/nA) Breakdown at high reverse voltage Animation: Draw the P and N blocks with holes and electrons. Show diffusion forming the depletion region. Animate forward bias: the region narrows and carriers cross. Reverse bias: the region widens and current stops. AI image/animation prompt: PN junction with holes and electrons, depletion region narrowing under forward bias and widening under reverse bias, whiteboard style, 16:9 | A diode is a PN junction. A P-type semiconductor has an excess of holes, and an N-type has an excess of free electrons. When they are joined, electrons and holes near the junction diffuse across and recombine, leaving a region with no free carriers, called the depletion region. It sets up a barrier potential: about 0.7 volts for silicon, and 0.3 volts for germanium. In forward bias, the positive terminal of the supply goes to the P side. This reduces the barrier, the depletion region narrows, and once the applied voltage exceeds the barrier potential, a large current flows. In reverse bias, the positive terminal is connected to the N side. The barrier grows, the depletion region widens, and only a tiny leakage current flows. So the diode conducts in one direction, and blocks in the other. [137 words ≈ 59 sec] |
| 1:35 – 2:37 SCENE 2: V-I CHARACTERISTIC Visual: Diode I-V graph with forward region, knee at 0.7 V, reverse region and breakdown. On-screen text: Shockley: I = Is (e^(V/(n VT)) − 1) , VT ≈ 26 mV at 300 K Forward: negligible current until the knee (≈ 0.7 V Si), then rises steeply Reverse: I ≈ −Is (tiny) Breakdown at −VBR Dynamic resistance rd = n VT / ID ≈ 26 mV / ID Animation: Plot the characteristic with different scales for forward and reverse (mA vs µA). Highlight the knee at 0.7 V. Zoom into a small portion to show rd as the inverse slope. AI image/animation prompt: Diode I-V characteristic curve with knee at 0.7 volts, reverse leakage current and breakdown, whiteboard style, 16:9 | The V-I characteristic shows this behaviour. In the forward direction, the current is negligible until the voltage reaches the knee, about 0.7 volts for silicon, and then it rises very steeply. In the reverse direction, the current is a tiny leakage, microamperes or less, almost constant until a large reverse voltage causes breakdown. Mathematically, the Shockley equation gives I equal to I s into e to the power V by n V T, minus 1. Here V T is the thermal voltage, about 26 millivolts at room temperature. The slope of the curve at an operating point gives the dynamic resistance, r d equals 26 millivolts divided by the diode current. At 10 milliamperes, r d is only about 2.6 ohm. Temperature matters: the barrier potential falls by about 2 millivolts for every degree Celsius rise. That is why a hot diode drops slightly less voltage. [146 words ≈ 63 sec] |
| 2:37 – 3:29 SCENE 3: DIODE EQUIVALENT CIRCUITS Visual: Three equivalent circuits: ideal switch; switch + 0.7 V battery; switch + battery + rd. On-screen text: 1. Ideal: ON = short circuit (0 V) ; OFF = open circuit 2. Simplified: ON = 0.7 V battery in series ; OFF = open 3. Piecewise-linear: ON = 0.7 V battery + rd in series ; OFF = open Test: assume ON, calculate ID; if ID > 0 → ON valid; else OFF Animation: Show the real V-I curve, and overlay each approximation as straight lines: step at zero, step at 0.7 V, sloped line at 0.7 V. Draw the matching small circuits beside each. AI image/animation prompt: Three diode equivalent circuits next to the real V-I curve approximated by ideal step, shifted step at 0.7 volts and sloped line, whiteboard style, 16:9 | A circuit with a diode is non-linear. To make analysis easy, we replace the diode with an equivalent circuit, depending on how accurate we need to be. The ideal diode is a perfect switch: a short circuit when conducting, and an open circuit when blocking. The simplified model adds a 0.7 volt drop in series with the ON switch. And the piecewise-linear model adds the small dynamic resistance as well. For a circuit with several diodes, assume a state, ON or OFF, for each diode, calculate, and then check. If an assumed ON diode has a positive current, the assumption is right. If an assumed OFF diode has a reverse voltage across it, that is also consistent. Otherwise change the assumption. [121 words ≈ 52 sec] |
| 3:29 – 4:50 SOLVED NUMERICAL Visual: Silicon diode in series with R = 1 kΩ and a 10 V source (forward biased). Find ID using each model; datasheet rd = 10 Ω. On-screen text: Ideal: ID = 10/1000 = 10 mA , VD = 0 Simplified: ID = (10 − 0.7)/1000 = 9.3 mA , VD = 0.7 V Piecewise-linear (rd = 10 Ω): ID = (10 − 0.7)/(1000 + 10) = 9.21 mA , VD = 0.7 + 9.21 mA × 10 Ω = 0.792 V VR = 9.21 V Reverse connected: ID ≈ 0 , VR = 0 , Vdiode ≈ −10 V rd (from 26 mV/ID at 9.3 mA) = 2.8 Ω Animation: Draw the circuit with the forward-biased diode. Replace the diode by each model in turn, and compute the current. Then flip the diode and show zero current with the full 10 V across the diode. AI image/animation prompt: Series circuit with 10 volt source 1 kilo ohm resistor and diode, replaced by three equivalent circuits with calculated currents 10 mA 9.3 mA 9.21 mA, and reverse biased case with zero current, 16:9 | A silicon diode is in series with a 1 kilo-ohm resistor and a 10 volt source, forward biased. Find the diode current using the three models. Assume r d equals 10 ohm from the datasheet for the piecewise-linear model. Ideal diode. The diode is a short circuit. The current equals 10 volts divided by 1 kilo-ohm, equals 10 milliamperes. Simplified model. The diode drops 0.7 volts. The current equals 10 minus 0.7, divided by 1,000, equals 9.3 milliamperes. Piecewise-linear model. The total series resistance is 1,010 ohm. The current equals 9.3 divided by 1,010, equals 9.21 milliamperes. The diode voltage equals 0.7 plus 9.21 milliamperes into 10 ohm, equals 0.792 volts. See how close the answers are: 10, 9.3 and 9.21. The 0.7 volt drop mattered by 7 percent here. But for a 3 volt source, it would matter a great deal. So use the ideal model only when the source is much larger than 0.7 volts. Now reverse the diode. It is reverse biased, so it blocks. The current is almost zero, the voltage across the resistor is zero, and the entire 10 volts appears across the diode. [189 words ≈ 81 sec] |
| 4:50 – 7:02 COMMON MISTAKES AND EXAM TIPS Visual: Diode mistakes. On-screen text: 1. Check the polarity: arrow = conventional current direction 2. Si 0.7 V, Ge 0.3 V; LED about 2 V 3. Ideal model only if V >> 0.7 V 4. Always verify the assumed ON/OFF state Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: the diode symbol arrow points in the direction of the conventional current when it conducts, from anode to cathode. Two: remember the drops: 0.7 volts for silicon, 0.3 for germanium. LEDs have a larger drop, typically 1.8 to 3 volts depending on the colour. Three: use the ideal model only when the supply is much larger than the diode drop. Four: always verify the assumed state. Real exam questions with several diodes are often solved by trial. Assume, compute, and check for contradictions. Real-life use: besides rectifiers, diodes protect circuits from reverse-polarity batteries, clip signals, and prevent the back-flow of current. A small relay coil, for example, uses a diode across it to absorb the voltage spike when the coil is switched off. Before we finish, a few practical items. When selecting a diode, read the data sheet. The important numbers are the maximum average forward current, the peak inverse voltage or reverse breakdown rating, and the forward voltage drop. For example, the general-purpose rectifier diode 1N4007 is rated about 1 ampere and 1000 volts, while the small-signal 1N4148 handles about 200 milliamperes at 100 volts, and switches very fast. There are also special diodes. The light-emitting diode emits light when forward biased, with a forward drop of about 2 volts, and always needs a series resistor. The photodiode works the opposite way: light produces a reverse current. The Schottky diode has a low forward drop, about 0.3 volts, and switches quickly, so it is used in power supplies. And in the next videos, we will meet the Zener diode. One simple application right now: a clipper. A diode with a resistor can limit a signal to about 0.7 volts, protecting an input from excessive voltage. Try drawing the output for a sine input, and you will see its positive peaks flattened at 0.7 volts. [308 words ≈ 132 sec] |
| 7:02 – 7:29 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. PN junction: depletion region, barrier ≈ 0.7 V (Si) 2. Forward bias conducts; reverse bias blocks 3. I = Is(e^(V/nVT) − 1) 4. Models: ideal, 0.7 V, 0.7 V + rd 5. Assume ON/OFF, calculate, verify Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: a PN junction has a depletion region and a barrier potential of about 0.7 volts for silicon Two: forward bias conducts, and reverse bias blocks Three: the Shockley equation describes the V-I characteristic Four: the three equivalent circuits are ideal, 0.7 volt, and 0.7 volt with dynamic resistance Five: assume the state, calculate, and verify [62 words ≈ 27 sec] |
| 7:29 – 8:13 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: Si diode, 5 V source, 470 Ω resistor, forward biased. Find ID (simplified model). Repeat for Ge (0.3 V). Practice 2: If the diode above is reversed, what are ID and the voltage across R? Next: Half-Wave and Full-Wave Bridge Rectifiers Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A sine wave being converted to a pulsating DC waveform by a bridge of four diodes, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a silicon diode is in series with a 470 ohm resistor and a 5 volt source, forward biased. Find the current using the simplified model. Repeat for germanium. Question two: if the diode above is reversed, what are the current and the voltage across the resistor? Answers are in the pinned comment. In the next video, we use diodes to convert AC to DC in half-wave and full-wave bridge rectifiers. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [104 words ≈ 45 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Si: (5 − 0.7)/470 = 9.15 mA ; Ge: (5 − 0.3)/470 = 10 mA.

Practice 2 answer: ID ≈ 0 ; VR = 0 ; the diode has about 5 V reverse across it.

# YouTube package

### Video title

PN Junction Diode | V-I Characteristics & Equivalent Circuits (Ideal, Simplified, Piecewise Linear) | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the PN junction diode: depletion region, forward and reverse bias, the V-I characteristic, and the ideal, simplified and piecewise-linear equivalent circuits, with a solved numerical.

Chapters:

0:00 Hook

0:17 Learning objectives

0:36 The Pn Junction

1:35 V-I Characteristic

2:37 Diode Equivalent Circuits

3:29 Solved Numerical

4:50 Common Mistakes And Exam Tips

7:02 Recap

7:29 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

semiconductor diode, PN junction, diode V-I characteristics, diode equivalent circuit, ideal diode, silicon diode 0.7 V, forward bias reverse bias, basic electronics, BEEE, Prof Mitesh Astik

### Thumbnail text idea

DIODE  |  0.7 V  |  Ideal vs Practical Model

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits <-- THIS VIDEO Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |