# Half Wave & Full Wave Bridge Rectifier | Vdc, Vrms, Ripple Factor, PIV | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 3: Diodes, transistors and applications  |  Presenter: Prof. Mitesh Astik

Topic: Half-Wave and Full-Wave Bridge Rectifiers   |   Target length: 8:17 (1160 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:15 HOOK Visual: A wall socket, a charger and a phone, with a waveform changing from sine to pulsating DC. On-screen text: AC in, DC out: four diodes Animation: Animate a sine wave going through a diode bridge and emerging as a series of positive humps, then smoothing out with a capacitor. AI image/animation prompt: Sine wave entering a four diode bridge and exiting as positive pulses, flat illustration, 16:9 | The power from your wall socket is AC, but your phone, laptop and television need DC. Inside every charger, a circuit of just four diodes converts one into the other. Let us see how. [34 words ≈ 15 sec] |
| 0:15 – 0:33 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 3: Diodes, transistors and applications By the end you can: 1. Half-wave 2. Bridge rectifier 3. Vdc, Vrms, ripple 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to explain half-wave rectification, explain the bridge rectifier, calculate Vdc, Vrms, ripple factor, and solve a numerical. Let us begin. [44 words ≈ 19 sec] |
| 0:33 – 1:52 SCENE 1: HALF-WAVE RECTIFIER Visual: Transformer secondary, diode, load RL; input and output waveforms. On-screen text: Positive half cycle: diode ON → output = input Negative half cycle: diode OFF → output = 0 Vdc = Vm/π = 0.318 Vm Vrms = Vm/2 Ripple factor γ = √[(Vrms/Vdc)² − 1] = 1.21 η = (Pdc/Pac) = 40.6 % (max) PIV = Vm Ripple frequency = f (50 Hz) Animation: Draw the circuit and the waveforms one under the other. Animate current flowing during the positive half cycle only. Show the Vdc as a dashed line at 0.318 Vm. List the results in a table. AI image/animation prompt: Half wave rectifier circuit with transformer diode and load, input sine wave above and output positive half cycles below, average value dashed, whiteboard style, 16:9 | The half-wave rectifier uses one diode in series with the load. During the positive half cycle, the diode is forward biased and conducts, so the output follows the input. During the negative half cycle, the diode is reverse biased, so the output is zero. The output is a series of positive half-sine pulses. The average value, which is the DC output, is the peak value divided by pi, equal to 0.318 V m. The RMS value is V m by 2. The ripple factor, a measure of the AC left in the output, is root of V rms by V dc whole squared, minus 1, which comes out to 1.21. This is greater than 1: the AC content is more than the DC! The maximum efficiency is only 40.6 percent. The diode must withstand a reverse voltage of V m, the peak inverse voltage. And the ripple frequency is the same as the supply frequency, 50 hertz. The half-wave rectifier is wasteful because it ignores half of the cycle. Its simplicity makes it suitable only for low-power uses such as small signal detectors. [183 words ≈ 78 sec] |
| 1:52 – 2:56 SCENE 2: FULL-WAVE BRIDGE RECTIFIER Visual: Four diodes D1–D4 in a bridge; current paths in positive and negative half cycles. On-screen text: Positive half cycle: D1 and D3 conduct Negative half cycle: D2 and D4 conduct Current through the load is always in the same direction Vdc = 2Vm/π = 0.636 Vm Vrms = Vm/√2 = 0.707 Vm Ripple factor γ = 0.482 η = 81.2 % PIV = Vm (each diode) Ripple frequency = 2f (100 Hz) With diode drops: Vdc ≈ 2(Vm − 1.4)/π Animation: Draw the bridge. Animate the current path in blue for the positive half cycle (D1, D3), and in red for the negative half cycle (D2, D4). Show the output as a train of positive humps with the DC level dashed. AI image/animation prompt: Four diode bridge rectifier with current path highlighted in blue for positive half cycle and red for negative, output waveform of positive humps with average line, 16:9 | The full-wave bridge uses four diodes and no centre-tapped transformer. In the positive half cycle, diodes D1 and D3 conduct, and the current passes through the load from top to bottom. In the negative half cycle, D2 and D4 conduct, and the current again passes through the load in the same direction. The output has two humps for every input cycle. The average value doubles to 2 V m by pi, equal to 0.636 V m. The RMS value is V m by root 2. The ripple factor falls to 0.482, the efficiency rises to 81.2 percent, and the ripple frequency doubles to 100 hertz, which makes filtering easier. The peak inverse voltage for each diode is V m. In a real bridge, two diodes are in the path, so the output loses about 1.4 volts, and V dc is 2 times V m minus 1.4, divided by pi. [149 words ≈ 64 sec] |
| 2:56 – 3:46 SCENE 3: FILTERING AND PRACTICAL POINTS Visual: Bridge output with a smoothing capacitor; the ripple reducing. On-screen text: A capacitor filter across RL charges to Vm and discharges slowly Ripple ≈ Idc / (2 f C) for full-wave (peak-to-peak approximation) Larger C or larger RL → less ripple Transformer provides isolation and suitable voltage Animation: Animate the capacitor charging on the peaks and discharging between them, smoothing the humps. Show two capacitor values: small (large ripple) and large (small ripple). AI image/animation prompt: Rectified waveform with smoothing capacitor showing charging at peaks and slow discharge between, comparing small and large capacitor values, whiteboard style, 16:9 | The rectified output, though always positive, is still pulsating. To obtain a steady DC, we connect a large capacitor across the load. It charges to the peak voltage, and discharges slowly into the load between the peaks. A bigger capacitor, or a lighter load, means less ripple. Because the full-wave rectifier has peaks twice as often, the capacitor discharges for less time, and the ripple is smaller than for a half-wave rectifier with the same capacitor. In a real power supply, a transformer comes first, to step the voltage down and to isolate the DC side from the mains. Then comes the bridge, the filter capacitor, and often a regulator, which we study in the next video. [117 words ≈ 50 sec] |
| 3:46 – 5:34 SOLVED NUMERICAL Visual: Transformer secondary 18 V rms. Load RL = 100 Ω. Find Vdc and Idc for HWR and bridge (ideal diodes), the bridge with 0.7 V diodes, ripple factors and PIV. On-screen text: Vm = 18√2 = 25.46 V HWR: Vdc = Vm/π = 8.10 V ; Idc = 81 mA ; Vrms = Vm/2 = 12.73 V ; γ = √[(12.73/8.10)² − 1] = 1.21 ; PIV = 25.46 V Bridge (ideal): Vdc = 2Vm/π = 16.2 V ; Idc = 162 mA ; Vrms = 18 V ; γ = √[(18/16.2)² − 1] = 0.483 ; PIV = 25.46 V Bridge (0.7 V diodes): Vdc = 2(25.46 − 1.4)/π = 15.31 V ; Idc = 153 mA Animation: Draw the bridge circuit with values. Calculate step by step with a table comparing half-wave, bridge ideal and bridge practical. Show the output waveforms next to each result. AI image/animation prompt: Table comparing half wave rectifier and bridge rectifier results for 18 V rms secondary with peak voltage 25.46 and DC voltages 8.10 16.2 and 15.31, output waveforms beside, 16:9 | A transformer secondary gives 18 volts RMS to a rectifier feeding a 100 ohm load. Find the DC output voltage and current for a half-wave rectifier, and a bridge rectifier. Also find the ripple factors, the peak inverse voltage, and the effect of real diode drops. Step one: the peak voltage. V m equals 18 into root 2, equals 25.46 volts. Half-wave. V dc equals V m by pi, equals 8.10 volts. The DC current equals 81 milliamperes. V rms equals V m by 2, equals 12.73 volts. The ripple factor equals root of 12.73 by 8.10 whole squared, minus 1. That is root of 2.47 minus 1, which is 1.21. The PIV is 25.46 volts. Bridge, ideal diodes. V dc equals 2 V m by pi, equals 16.2 volts. The DC current is 162 milliamperes. V rms equals 18 volts. The ripple factor equals root of 18 by 16.2 whole squared, minus 1, equals root of 0.234, equals 0.483. The PIV is again 25.46 volts, per diode. Bridge, with silicon diodes. Two diodes are in series with the load at any time, so we subtract 1.4 volts from the peak. V dc equals 2 into 25.46 minus 1.4, divided by pi. That is 2 into 24.06, divided by pi, equals 15.31 volts. The current is 153 milliamperes. So a real bridge gives about 5 percent less than the ideal. The bridge rectifier gives twice the DC output of the half-wave circuit, and less than half its ripple, from the same transformer. [252 words ≈ 108 sec] |
| 5:34 – 6:56 COMMON MISTAKES AND EXAM TIPS Visual: Rectifier mistakes. On-screen text: 1. Vm = √2 × Vrms 2. Bridge: 2 diodes drop in series (1.4 V) 3. Ripple factor uses Vrms/Vdc of the OUTPUT 4. PIV of bridge diode is Vm Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: always find the peak value first, V m equals root 2 times the RMS voltage of the secondary. Two: in a bridge, two diodes conduct at the same time, so subtract two drops, 1.4 volts, from the peak. Three: for the ripple factor, use the output RMS and output DC values, not the input values. Four: the peak inverse voltage of each diode in the bridge is V m. In a centre-tapped full-wave rectifier, it is 2 V m, which is why the bridge is preferred for high voltages. Real-life use: the same bridge is inside nearly every charger, adapter and power supply. When you see a small black cube with four legs, it is a complete bridge rectifier in one package. For comparison, there is another full-wave circuit, the centre-tapped rectifier. It uses only two diodes, but needs a transformer with a centre tap, and each diode must withstand a peak inverse voltage of 2 V m. The bridge needs four diodes, but no centre tap, and each diode withstands only V m. For this reason, the bridge is the more common choice, especially at higher voltages. [191 words ≈ 82 sec] |
| 6:56 – 7:32 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. HWR: Vdc = Vm/π ; γ = 1.21 ; η = 40.6 % ; PIV = Vm 2. Bridge: Vdc = 2Vm/π ; γ = 0.482 ; η = 81.2 % 3. Bridge: D1, D3 / D2, D4 alternately; ripple 100 Hz 4. Real diodes: subtract 1.4 V in the bridge 5. Capacitor filter reduces ripple Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the half wave rectifier gives V m by pi, a ripple factor of 1.21, efficiency 40.6 percent, and PIV equal to V m Two: the bridge gives 2 V m by pi, a ripple factor of 0.482, and efficiency 81.2 percent Three: the bridge conducts through D1 D3 and D2 D4 alternately, and the ripple frequency is twice the supply Four: with real diodes subtract about 1.4 volts in the bridge Five: a capacitor filter reduces the ripple [85 words ≈ 36 sec] |
| 7:32 – 8:17 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: Bridge rectifier, secondary 12 V rms, ideal diodes. Find Vdc. Practice 2: HWR with Vm = 10 V. Find Vdc, Vrms and the ripple frequency (50 Hz supply). Next: Zener Diode as a Voltage Regulator Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A Zener diode holding a steady voltage while the supply wobbles, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a bridge rectifier has a 12 volt RMS secondary with ideal diodes. Find the DC output voltage. Question two: a half wave rectifier has a peak voltage of 10 volts. Find V dc, V rms and the ripple frequency for a 50 hertz supply. Answers are in the pinned comment. In the next video, we use the Zener diode to keep the DC output voltage constant, and design a simple regulator. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [105 words ≈ 45 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Vm = 16.97 V ; Vdc = 2×16.97/π = 10.80 V.

Practice 2 answer: Vdc = 3.18 V ; Vrms = 5 V ; ripple frequency = 50 Hz.

# YouTube package

### Video title

Half Wave & Full Wave Bridge Rectifier | Vdc, Vrms, Ripple Factor, PIV | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains half-wave and full-wave bridge rectifiers: operation, average and RMS output, ripple factor, efficiency, and peak inverse voltage, with a worked numerical.

Chapters:

0:00 Hook

0:15 Learning objectives

0:33 Half-Wave Rectifier

1:52 Full-Wave Bridge Rectifier

2:56 Filtering And Practical Points

3:46 Solved Numerical

5:34 Common Mistakes And Exam Tips

6:56 Recap

7:32 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

half wave rectifier, full wave bridge rectifier, ripple factor, PIV peak inverse voltage, rectifier efficiency, average DC output, AC to DC conversion, basic electronics, BEEE, Prof Mitesh Astik

### Thumbnail text idea

RECTIFIER  |  HWR 40.6%  |  BRIDGE 81.2%  |  Ripple 1.21 vs 0.48

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers <-- THIS VIDEO Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |