# n-Channel Enhancement MOSFET | Working, Regions, Characteristics | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 4: Transistors  |  Presenter: Prof. Mitesh Astik

Topic: n-Channel Enhancement MOSFET   |   Target length: 8:46 (1228 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:18 HOOK Visual: A microchip zooming in to a single MOSFET transistor. On-screen text: Billions of switches on one chip Animation: Animate a zoom from a phone to a chip to a single transistor cross-section, with the channel forming as a gate voltage is applied. AI image/animation prompt: Zoom sequence from smartphone to microchip to a MOSFET cross-section with gate drain source and a forming channel, flat illustration, 16:9 | Your phone has billions of transistors on a chip smaller than your fingernail. They are almost all MOSFETs. Why this one? Because it needs almost no input current, and it can be made extremely small. Let us see how it works. [41 words ≈ 18 sec] |
| 0:18 – 0:38 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 4: Transistors By the end you can: 1. Construction 2. Channel and Vth 3. Region equations 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to describe MOSFET construction, explain channel formation and threshold voltage, write drain current equations in each region, and solve a numerical. Let us begin. [48 words ≈ 21 sec] |
| 0:38 – 1:37 SCENE 1: CONSTRUCTION AND CHANNEL FORMATION Visual: Cross-section: p-type substrate, n+ source and drain, thin SiO2 oxide, metal/poly gate; channel forming under the gate. On-screen text: p-type substrate; n+ Source and Drain; thin SiO₂ insulator; Gate on top VGS = 0: no channel → OFF (enhancement type) VGS > Vth: electrons attracted to the surface → n-channel (inversion layer) → conduction Gate is insulated → IG ≈ 0 (very high input resistance) Vth ≈ 1–2 V (typical discrete) Animation: Draw the cross-section. Animate VGS rising: positive charges on the gate repel holes and attract electrons, forming a blue channel between source and drain once VGS passes Vth. AI image/animation prompt: MOSFET cross-section with p substrate n plus source and drain oxide layer and gate, electrons forming an inversion channel as gate voltage exceeds threshold, whiteboard style, 16:9 | The n-channel enhancement MOSFET is built on a p-type substrate, with two heavily doped n regions: the source and the drain. Above the region between them is a very thin layer of silicon dioxide, an insulator, and above that the metal gate. Since the gate is insulated, essentially no current flows into it, so the input resistance is extremely high. With zero voltage on the gate, there is no connection between source and drain; two back-to-back diodes block the current. This is why it is called an enhancement type. As a positive voltage is applied to the gate, it attracts electrons to the surface under the oxide, and repels holes. Once V G S exceeds the threshold voltage, V th, enough electrons gather to form a thin n-type channel connecting the source and drain. Current can now flow. [138 words ≈ 59 sec] |
| 1:37 – 3:09 SCENE 2: OPERATING REGIONS AND CHARACTERISTICS Visual: Drain characteristics ID vs VDS for several VGS; cutoff, triode and saturation regions; the boundary VDS = VGS − Vth. On-screen text: Cutoff: VGS < Vth → ID = 0 Triode (ohmic): VGS > Vth , VDS < VGS − Vth → ID = k[(VGS − Vth)VDS − VDS²/2] Saturation: VGS > Vth , VDS ≥ VGS − Vth → ID = (k/2)(VGS − Vth)² k = µn·Cox·(W/L) Triode: acts as a voltage-controlled resistor ; Saturation: amplifier Animation: Plot the family of curves, shading the triode region (ID rising with VDS) and saturation region (flat). Draw the dashed boundary parabola VDS = VGS − Vth. Mark the transfer characteristic ID vs VGS as a parabola starting at Vth. AI image/animation prompt: MOSFET drain characteristics with triode and saturation regions, boundary parabola, and transfer characteristic starting at threshold voltage, whiteboard style, 16:9 | The behaviour depends on V G S and V D S. If V G S is below V th, there is no channel and the drain current is zero: cutoff. If V G S is above V th and V D S is small, the channel acts like a resistor, and the current rises with V D S. This is the triode or ohmic region, valid when V D S is less than V G S minus V th. As V D S increases, the channel gets pinched near the drain. When V D S reaches V G S minus V th, the current stops rising, and the device enters saturation. In saturation, the drain current is k by 2 times V G S minus V th, squared. It depends only on V G S, not on V D S. The constant k depends on the process and the width to length ratio of the channel. In the triode region, the expression includes V D S, and the MOSFET behaves as a voltage-controlled resistor. For switching, the MOSFET works in cutoff and triode. For amplifiers, it works in saturation. In saturation, the real device has a slight increase of current with V D S, called channel-length modulation. We neglect it in this course. [213 words ≈ 91 sec] |
| 3:09 – 3:49 SCENE 3: MOSFET VS BJT Visual: Comparison table. On-screen text: MOSFET: voltage-controlled, IG ≈ 0 , very high input resistance, small size, low power, easy to integrate BJT: current-controlled, input current IB, lower input resistance, higher gm for the same current, more power, larger Both: switch, amplifier Handling: MOSFET gate is sensitive to static discharge Animation: Show the two devices side by side with a comparison table appearing row by row. AI image/animation prompt: Side by side comparison table of MOSFET and BJT with symbols, flat style, 16:9 | How does the MOSFET compare with the BJT? The MOSFET is voltage controlled, with almost no input current and a very high input resistance. It is small, uses little power, and is simple to fabricate in huge numbers. That is why it dominates digital integrated circuits. The BJT is current controlled and needs a base current, but it gives a higher gain for the same current and is still widely used in analogue circuits. A practical caution: the thin oxide layer can be damaged by static electricity. Handle MOSFETs with care, and use anti-static precautions. [95 words ≈ 41 sec] |
| 3:49 – 6:20 SOLVED NUMERICAL Visual: n-MOSFET with k = 2 mA/V², Vth = 2 V. (a) ID for VGS = 3, 4 V. (b) In a circuit with VDD = 10 V, RD = 1.5 kΩ, find the operating point for VGS = 4 V and VGS = 6 V. On-screen text: (a) ID = (k/2)(VGS − Vth)² = 1 mA/V² × (VGS − 2)² VGS = 3 V → 1 mA ; VGS = 4 V → 4 mA (b) VGS = 4 V: ID(sat) = 4 mA → VDS = 10 − 1.5 k × 4 mA = 4 V ≥ VGS − Vth = 2 V → saturation ✓ VGS = 6 V: saturation would need ID = 16 mA → VDS < 0 ✗ → triode ID = (10 − VDS)/1.5k = 2 m[4VDS − VDS²/2] → 1.5VDS² − 13VDS + 10 = 0 VDS = (13 − √109)/3 = 0.853 V , ID = 6.10 mA ✓ (VDS < VGS − Vth = 4 V) Animation: Draw the circuit with the load line. For VGS = 4 V show the saturation point; for VGS = 6 V first test the saturation assumption (fails), then solve the quadratic and mark the triode operating point on the load line. AI image/animation prompt: MOSFET with drain resistor and 10 volt supply, load line on drain characteristics with operating points for gate voltage 4 volts in saturation and 6 volts in triode, whiteboard style, 16:9 | A MOSFET has k equal to 2 milliampere per volt squared, and a threshold voltage of 2 volts. Part A: find the drain current in saturation for V G S equal to 3 and 4 volts. Part B: in a circuit with V D D equal to 10 volts and a 1.5 kilo-ohm drain resistor, find the operating point for V G S equal to 4 volts, and for 6 volts. Part A. I D equals k by 2, which is 1 milliampere per volt squared, times V G S minus 2, squared. At V G S equal to 3, I D equals 1 milliampere. At 4 volts, I D equals 4 milliamperes. Notice that doubling the overdrive, from 1 to 2 volts, quadruples the current. Part B, V G S equal to 4 volts. Assume saturation: I D is 4 milliamperes. The drop across the drain resistor is 1.5 kilo-ohm times 4 milliampere, equals 6 volts, so V D S equals 4 volts. Check: V G S minus V th is 2 volts, and V D S of 4 is greater, so saturation holds. V G S equal to 6 volts. Assume saturation: I D would be 1 times 4 squared, equals 16 milliamperes. The drop across the resistor would be 24 volts, more than the supply, so V D S would be negative. Impossible, so the device is in the triode region. In the triode region, I D equals 2 milliamperes times 4 V D S minus V D S squared by 2. And I D also equals 10 minus V D S, divided by 1.5 kilo-ohm. Equating, and multiplying by 1,500: 10 minus V D S equals 12 V D S minus 1.5 V D S squared. So 1.5 V D S squared minus 13 V D S plus 10 equals 0. The roots are 0.853 and 7.8; the smaller one is valid, because it satisfies V D S less than 4. So V D S equals 0.853 volts, and I D equals 6.1 milliamperes. The MOSFET is acting almost like a closed switch, with a small drop. [351 words ≈ 150 sec] |
| 6:20 – 7:19 COMMON MISTAKES AND EXAM TIPS Visual: MOSFET mistakes. On-screen text: 1. Always check the assumed region 2. Saturation condition VDS ≥ VGS − Vth 3. Square the overdrive (VGS − Vth) 4. Discard the root that violates the region Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: always check the assumed region after solving. If you assume saturation, you must verify V D S is at least V G S minus V th. Two: in saturation, the current depends on the square of the overdrive voltage, V G S minus V th. Three: when solving quadratics in the triode region, discard the root that violates the region condition, as we did. Four: do not confuse MOSFET saturation with BJT saturation. In a BJT, saturation means the switch is ON. In a MOSFET, saturation means the current is constant, which is the amplifier region. For a MOSFET switch, the ON state is the triode region. Real-life use: power MOSFETs switch heavy loads, like motors and LED lighting, with very small losses, and the MOSFETs in a processor switch billions of times per second. [138 words ≈ 59 sec] |
| 7:19 – 7:55 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. MOSFET: insulated gate, IG ≈ 0, channel forms for VGS > Vth 2. Cutoff: VGS < Vth 3. Triode: VDS < VGS − Vth ; Saturation: VDS ≥ VGS − Vth 4. Saturation: ID = (k/2)(VGS − Vth)² 5. MOSFET switch: cutoff/triode ; amplifier: saturation Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the MOSFET has an insulated gate, and a channel forms when V G S exceeds the threshold Two: below the threshold the MOSFET is in cutoff Three: triode and saturation are separated by V D S equal to V G S minus V th Four: in saturation the drain current is k by 2 times V G S minus V th squared Five: as a switch it uses cutoff and triode, and as an amplifier it uses saturation [85 words ≈ 36 sec] |
| 7:55 – 8:46 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: Same MOSFET (k = 2 mA/V², Vth = 2 V), VGS = 3.5 V in saturation. Find ID. Practice 2: For VGS = 1.5 V (Vth = 2 V), what is ID? In which region is the MOSFET? Next: Common-Source Amplifier, DC Load Line and MOS Inverter Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A MOSFET amplifier with a load line and a sine wave being amplified, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: same MOSFET with k equal to 2 milliampere per volt squared and V th equal to 2 volts, V G S equal to 3.5 volts in saturation. Find I D. Question two: for V G S equal to 1.5 volts and V th equal to 2 volts, what is I D, and in which region is the MOSFET? Answers are in the pinned comment. In the next video, we use the MOSFET as an amplifier with a DC load line, and as a digital inverter. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [119 words ≈ 51 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: ID = 1 mA/V² × (1.5)² = 2.25 mA.

Practice 2 answer: VGS < Vth → cutoff → ID = 0.

# YouTube package

### Video title

n-Channel Enhancement MOSFET | Working, Regions, Characteristics | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the construction, operation and characteristics of an n-channel enhancement MOSFET, the three regions, the drain current equations, and a solved numerical.

Chapters:

0:00 Hook

0:18 Learning objectives

0:38 Construction And Channel Formation

1:37 Operating Regions And Characteristics

3:09 Mosfet Vs Bjt

3:49 Solved Numerical

6:20 Common Mistakes And Exam Tips

7:19 Recap

7:55 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

MOSFET, n channel enhancement MOSFET, threshold voltage, triode region saturation region, MOSFET characteristics, drain current equation, MOS transistor, basic electronics, BEEE, Prof Mitesh Astik

### Thumbnail text idea

MOSFET  |  Vth  |  Triode vs Saturation  |  ID = (k/2)(VGS − Vth)²

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET <-- THIS VIDEO Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators |