# Op-Amp Integrator, Differentiator & Comparator | Waveforms & Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 5: Op-Amp and linear IC applications  |  Presenter: Prof. Mitesh Astik

Topic: Integrator, Differentiator and Comparator   |   Target length: 8:22 (1171 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:19 HOOK Visual: A square wave changing into a triangle wave and a sine wave being compared against a reference line. On-screen text: Calculus with a resistor and a capacitor Animation: Animate a square wave entering an integrator and exiting as a triangle; then a sine entering a comparator with a reference line and the output flipping between ±Vsat. AI image/animation prompt: Square wave to triangle wave through an integrator and a sine wave compared with a reference line producing a square output, flat illustration, 16:9 | Can a circuit do calculus? Yes. An op-amp with a resistor and a capacitor can integrate and differentiate signals in real time. And with no feedback at all, the op-amp becomes a comparator, deciding which of two voltages is higher. Let us see all three. [45 words ≈ 19 sec] |
| 0:19 – 0:38 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 5: Op-Amp and linear IC applications By the end you can: 1. Integrator 2. Differentiator 3. Comparator 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to derive the integrator output, derive the differentiator output, explain the comparator, and solve a numerical. Let us begin. [43 words ≈ 18 sec] |
| 0:38 – 1:47 SCENE 1: THE INTEGRATOR Visual: Inverting amplifier with the feedback resistor replaced by a capacitor C; waveform examples. On-screen text: Input R, feedback C; virtual ground at the inverting node I = Vin/R = C·d(0 − Vo)/dt → Vo = −(1/RC) ∫ Vin dt Constant input → ramp output: slope = −Vin/RC Square wave in → triangle out ; Sine in → cosine out (−90° shift) Practical: a large resistor across C limits DC drift Animation: Draw the circuit with the capacitor in the feedback path. Derive the output line by line. Animate waveforms: a constant step producing a ramp, a square wave producing a triangle. AI image/animation prompt: Op-amp integrator circuit with input resistor and feedback capacitor, with constant input giving a ramp output and square wave input giving a triangular output, whiteboard style, 16:9 | The integrator. Replace the feedback resistor of an inverting amplifier with a capacitor. The inverting input is still a virtual ground. The input current is V in by R. This current charges the capacitor, so it equals C times the rate of change of the voltage across it. The voltage across the capacitor is minus the output voltage. So V in by R equals minus C d V o by d t. Integrating, V o equals minus one over R C times the integral of V in d t. So the output is the integral of the input, scaled by 1 over R C. A constant input gives a ramp. A square wave gives a triangular wave. A sine wave gives a cosine wave, shifted by 90 degrees. Integrators are used in analogue computers, in waveform generators, and in analogue-to-digital converters. In practice, a high-value resistor across the capacitor prevents the output from drifting to saturation because of small DC offsets. [161 words ≈ 69 sec] |
| 1:47 – 2:46 SCENE 2: THE DIFFERENTIATOR Visual: Capacitor at the input, resistor in the feedback; waveform examples. On-screen text: Input C, feedback R; virtual ground I = C·dVin/dt = −Vo/R → Vo = −RC·dVin/dt Ramp in → constant out ; Triangle in → square out Sine in → cosine out ; Noise gets amplified (high-pass nature) Practical: add small series resistor with C and a small capacitor across R to limit noise Animation: Draw the circuit by swapping the positions of R and C. Show a triangle wave becoming a square wave. Show noise spikes appearing and the remedy network. AI image/animation prompt: Op-amp differentiator circuit with input capacitor and feedback resistor, triangle input giving square output, noise warning icon, whiteboard style, 16:9 | The differentiator swaps the positions: capacitor at the input and resistor in the feedback. The current through the capacitor is C times the rate of change of the input voltage. This flows through R f, so the output is minus R C times the derivative of the input. A triangular input produces a square output, and a ramp gives a constant. The problem with a differentiator is noise. Differentiation exaggerates fast changes, so high-frequency noise is greatly amplified. In practice, we add a small resistor in series with the capacitor and a small capacitor across the feedback resistor to limit the high-frequency gain. Because of this, integrators are preferred over differentiators in most designs. A rule of thumb: the integrator is a low-pass circuit, which smooths signals, while the differentiator is a high-pass circuit, which emphasises quick changes. [138 words ≈ 59 sec] |
| 2:46 – 3:42 SCENE 3: THE COMPARATOR Visual: Open-loop op-amp with Vin at + and Vref at −; output flipping between +Vsat and −Vsat; zero-crossing detector. On-screen text: No feedback → very high gain → saturation Vin > Vref → Vo = +Vsat ; Vin < Vref → Vo = −Vsat Vref = 0 → zero-crossing detector (sine → square) Swap inputs → inverting comparator Switching point: Vin = Vref Hysteresis (Schmitt trigger) to reduce noise chatter Animation: Draw the circuit and a sine wave with a horizontal reference line. Mark where they cross; the output square wave flips at each crossing. Show duty cycle changing as Vref moves. AI image/animation prompt: Op-amp comparator with sine input and reference voltage line, output square wave flipping at the crossings, duty cycle changing with reference, whiteboard style, 16:9 | The comparator uses the op-amp without feedback. With such a huge gain, even a tiny difference between the two inputs drives the output to saturation. If the signal at the non-inverting input is higher than the reference at the inverting input, the output goes to plus V sat. If it is lower, the output goes to minus V sat. The circuit compares the two voltages, and gives a digital-like output. With the reference set to zero, it is a zero-crossing detector, converting a sine wave into a square wave with the same frequency. Moving the reference changes the duty cycle of the output. One practical problem: if the input is noisy near the reference, the output flips repeatedly. A Schmitt trigger, which uses positive feedback to create two switching thresholds, solves this. [132 words ≈ 57 sec] |
| 3:42 – 5:45 SOLVED NUMERICAL Visual: (a) Integrator R = 10 kΩ, C = 0.1 µF, Vin = +2 V for 2 ms. (b) Differentiator R = 10 kΩ, C = 0.1 µF, Vin rising 2 V in 1 ms. (c) Comparator: Vref = 2 V, Vin = 3 sin(2π·50 t), ±Vsat = ±12 V. On-screen text: (a) RC = 10k × 0.1 µ = 1 ms ; Vo = −(1/RC)·Vin·t = −(2/1 ms) × 2 ms = −4 V (ramp −2 V/ms) (b) dVin/dt = 2 V/1 ms = 2000 V/s ; Vo = −RC·dVin/dt = −1 ms × 2000 = −2 V (c) Output +12 V when 3 sin θ > 2 → θ from sin⁻¹(2/3) = 41.8° to 138.2° Width = 96.4° → time = 96.4/360 × 20 ms = 5.35 ms of each 20 ms cycle (duty 26.8 %) Rising crossing at t = 41.8/360 × 20 = 2.32 ms Animation: For (a), show the ramp growing on a time axis to −4 V. For (b), show the input ramp and the flat output at −2 V. For (c), draw the sine and the 2 V line, shade the part above; mark the angles and show the output pulse of 5.35 ms. AI image/animation prompt: Integrator ramp output reaching minus 4 volts, differentiator constant output minus 2 volts, and sine wave with 2 volt reference line producing a 5.35 millisecond positive pulse, whiteboard style, 16:9 | Three quick problems. Part A: an integrator with R equal to 10 kilo-ohm and C equal to 0.1 microfarad, with a constant 2 volt input applied for 2 milliseconds. Part B: a differentiator with the same R and C, with an input that rises by 2 volts in 1 millisecond. Part C: a comparator with a 2 volt reference and a 3 volt peak, 50 hertz sine input, with saturation voltages of plus and minus 12 volts. Part A. R C equals 10 kilo-ohm into 0.1 microfarad, equals 1 millisecond. For a constant input, the output is minus V in by R C, times t. That is minus 2 volts by 1 millisecond, times 2 milliseconds, equals minus 4 volts. The output ramps down at 2 volts per millisecond. Part B. The input slope is 2 volts per millisecond, equal to 2,000 volts per second. The output equals minus R C times the slope, equals minus 1 millisecond into 2,000, equals minus 2 volts, constant during the ramp. Part C. The output is plus 12 volts when 3 sin theta exceeds 2, so sin theta exceeds 2 by 3. Inverse sine of 0.667 is 41.8 degrees. So the output is high from 41.8 degrees to 180 minus 41.8, which is 138.2 degrees. That is a width of 96.4 degrees. One cycle at 50 hertz takes 20 milliseconds, so the high time is 96.4 by 360 into 20, equals 5.35 milliseconds. The duty cycle is 26.8 percent. The output goes high at 2.32 milliseconds, falls at 7.68 milliseconds, and stays at minus 12 volts for the rest of the cycle. If the reference were zero, the duty cycle would be exactly 50 percent: the zero-crossing detector. [285 words ≈ 122 sec] |
| 5:45 – 7:08 COMMON MISTAKES AND EXAM TIPS Visual: Integrator / comparator mistakes. On-screen text: 1. Integrator output has a MINUS sign 2. Units: R·C in seconds 3. Comparator output is ±Vsat, not ±Vcc exactly 4. Check which input is signal and which is reference Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: do not forget the minus sign in the integrator and differentiator formulas. Two: use consistent units. R in ohms times C in farads gives seconds. Three: the saturation voltage of the comparator is slightly less than the supply, by a volt or two for a 741. Four: look carefully at which terminal has the signal. If the signal is on the inverting input, the output is inverted. A frequent viva question: why do we add a resistor across the capacitor in a practical integrator? The answer: without it, the capacitor offers no DC feedback, the op-amp has full open-loop gain at DC, and any small offset drives the output into saturation. Real-life use: integrators appear in analogue filters and in control loops, comparators in battery chargers, zero-crossing detectors in power circuits, and in every analogue-to-digital converter. One more practical point about the integrator. The ideal circuit has no DC feedback, so a small offset voltage will make the output ramp to saturation. A resistor across the capacitor provides DC feedback and limits the DC gain. The circuit then integrates properly only for frequencies well above one over 2 pi R f C. [195 words ≈ 84 sec] |
| 7:08 – 7:39 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. Integrator: Vo = −(1/RC)∫Vin dt 2. Square in → triangle out 3. Differentiator: Vo = −RC dVin/dt ; noisy 4. Comparator: Vin > Vref → +Vsat 5. Vref = 0 → zero-crossing detector Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the integrator output is minus one over R C times the integral of the input Two: an integrator turns a square wave into a triangle wave Three: the differentiator output is minus R C times the derivative, and it amplifies noise Four: the comparator output is plus V sat when the input exceeds the reference Five: with zero reference it is a zero crossing detector [72 words ≈ 31 sec] |
| 7:39 – 8:22 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: Integrator with R = 1 kΩ, C = 1 µF, Vin = 5 V for 1 ms. Find Vo. Practice 2: A zero-crossing detector has a sine input. What is the output waveform and duty cycle? Next: 555 Timer in Astable Mode and 78XX Voltage Regulators Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A 555 timer IC blinking an LED and a 7805 regulator chip, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: an integrator has R equal to 1 kilo-ohm and C equal to 1 microfarad, with input 5 volts for 1 millisecond. Find the output. Question two: a zero crossing detector has a sine input. What are the output waveform and duty cycle? Answers are in the pinned comment. In the next video, we finish the series with the 555 timer and the 78XX fixed voltage regulators. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [100 words ≈ 43 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: RC = 1 ms ; Vo = −(5/1 ms) × 1 ms = −5 V.

Practice 2 answer: A square wave swinging between ±Vsat, with 50 % duty cycle, at the same frequency as the input.

# YouTube package

### Video title

Op-Amp Integrator, Differentiator & Comparator | Waveforms & Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the op-amp integrator and differentiator, their waveforms, and the comparator with its output waveform, with a solved numerical.

Chapters:

0:00 Hook

0:19 Learning objectives

0:38 The Integrator

1:47 The Differentiator

2:46 The Comparator

3:42 Solved Numerical

5:45 Common Mistakes And Exam Tips

7:08 Recap

7:39 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

op amp integrator, op amp differentiator, op amp comparator, zero crossing detector, square wave to triangle, integrator output waveform, operational amplifier applications, analog computer, BEEE, Prof Mitesh Astik

### Thumbnail text idea

INTEGRATOR  Vo = −(1/RC)∫Vin dt  |  COMPARATOR  |  Square → Triangle

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator <-- THIS VIDEO Ep21. 555 timer astable mode and 78XX voltage regulators |