# 555 Timer Astable Multivibrator & 78XX Voltage Regulator | Frequency, Duty Cycle | Solved Numerical | BEEE

Basic Electrical and Electronics Engineering  |  Unit 5: Op-Amp and linear IC applications  |  Presenter: Prof. Mitesh Astik

Topic: 555 Timer in Astable Mode and 78XX Voltage Regulators   |   Target length: 8:44 (1223 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:23 HOOK Visual: A 555 timer chip blinking an LED and a 7805 regulator chip next to it. On-screen text: Two famous chips: 555 and 7805 Animation: Animate a breadboard with a 555 chip flashing an LED and a three-pin 7805 regulator providing a steady 5 V line. AI image/animation prompt: 555 timer chip blinking an LED on a breadboard beside a three pin 7805 voltage regulator, flat illustration, 16:9 | A blinking LED on a toy, a siren, a clock pulse for a circuit: all of these can be built around one eight-pin chip invented in the early 1970s, the 555 timer. And the steady 5 volts that powers your microcontroller often comes from another famous three-pin chip, the 7805. Let us learn both. [54 words ≈ 23 sec] |
| 0:23 – 0:42 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Basic Electrical and Electronics Engineering ¦ Unit 5: Op-Amp and linear IC applications By the end you can: 1. 555 astable 2. f and duty cycle 3. 78XX regulators 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Basic Electrical and Electronics Engineering. By the end of this video, you will be able to explain the 555 astable circuit, calculate frequency and duty cycle, describe 78XX regulators, and solve a numerical. Let us begin. [45 words ≈ 19 sec] |
| 0:42 – 1:31 SCENE 1: INSIDE THE 555 TIMER Visual: Block diagram: three 5 kΩ resistors (voltage divider), two comparators, SR flip-flop, discharge transistor, output stage. On-screen text: Three equal 5 kΩ resistors → reference levels: 2/3 VCC (upper) and 1/3 VCC (lower) Upper comparator (threshold, pin 6): sets RESET when V > 2/3 VCC Lower comparator (trigger, pin 2): sets SET when V < 1/3 VCC Flip-flop output (pin 3) ; Discharge transistor (pin 7) Pin 8 VCC , Pin 1 GND , Pin 4 reset , Pin 5 control Animation: Draw the internal block diagram and label the eight pins. Highlight the voltage divider levels 1/3 and 2/3 VCC. Show the flip-flop toggling when the comparators trigger. AI image/animation prompt: Internal block diagram of the 555 timer with three resistor divider two comparators flip flop and discharge transistor, eight pins labelled, whiteboard style, 16:9 | The 555 timer gets its name from three 5 kilo-ohm resistors in a voltage divider inside it. They create two reference levels: two thirds of the supply and one third of the supply. Two comparators watch an external capacitor voltage. The threshold comparator on pin 6 senses when the voltage exceeds two thirds of V C C. The trigger comparator on pin 2 senses when it falls below one third. The comparators set and reset a flip-flop whose output drives pin 3. A transistor connected to pin 7 discharges the capacitor when the output is low. With just two resistors and a capacitor outside, this arrangement creates a continuous square wave: the astable mode. [114 words ≈ 49 sec] |
| 1:31 – 2:51 SCENE 2: ASTABLE MODE: OPERATION Visual: Astable circuit: R1 from VCC to pin 7; R2 between pin 7 and pins 2–6; capacitor C to ground; capacitor voltage waveform between 1/3 and 2/3 VCC and the output square wave. On-screen text: Pins 2 and 6 joined to the capacitor Charging: through R1 + R2 : V rises from 1/3 to 2/3 VCC → output HIGH tH = 0.693 (R1 + R2) C Discharging: through R2 into pin 7 : V falls from 2/3 to 1/3 VCC → output LOW tL = 0.693 R2 C T = tH + tL = 0.693 (R1 + 2R2) C f = 1.44 / [(R1 + 2R2) C] Duty cycle D = tH/T = (R1 + R2)/(R1 + 2R2) > 50 % Animation: Draw the circuit. Animate the capacitor charging along the exponential curve towards VCC but being stopped at 2/3 VCC; the discharge transistor turns on and the capacitor discharges to 1/3 VCC, and so on. Show the output square wave beneath, aligned in time. AI image/animation prompt: 555 astable circuit with R1 R2 and C and capacitor voltage sawtooth like curve between one third and two thirds of supply with the output square wave aligned below, whiteboard style, 16:9 | In astable mode, pins 2 and 6 are tied together and connected to the capacitor. When the power is switched on, the capacitor charges through R 1 plus R 2. While its voltage is below two thirds of the supply, the output is high. When it reaches two thirds, the upper comparator resets the flip-flop, the output goes low, and the discharge transistor turns on. Now the capacitor discharges through R 2 into pin 7. When its voltage falls to one third of the supply, the lower comparator sets the flip-flop, and the cycle repeats. The high time is 0.693 times R 1 plus R 2, times C. The low time is 0.693 times R 2 times C. The total period is 0.693 times R 1 plus 2 R 2, times C. So the frequency is 1.44 divided by R 1 plus 2 R 2, times C. The duty cycle is R 1 plus R 2, divided by R 1 plus 2 R 2, always greater than 50 percent in this simple circuit. To get close to 50 percent, make R 1 much smaller than R 2. [187 words ≈ 80 sec] |
| 2:51 – 4:02 SCENE 3: 78XX FIXED VOLTAGE REGULATORS Visual: Three-terminal regulator: input, ground, output; the 7805 with input and output capacitors; the series of 7805, 7806, 7808, 7812, 7815. On-screen text: 78XX: positive fixed output ; XX = output voltage (05 → 5 V , 12 → 12 V , 15 → 15 V) Three pins: Input – Ground (common) – Output Input must exceed output by the dropout (≈ 2 V) : 7805 needs Vin ≥ 7 V (max ≈ 35 V) Internal: reference, error amplifier, series pass transistor, thermal and short-circuit protection Capacitors: 0.33 µF at input, 0.1 µF at output Power dissipation Pd = (Vin − Vout) × IL → heat sink if large 79XX: negative series Animation: Draw the three-pin package with a block diagram inside. Show the typical connection with capacitors. Animate a varying input voltage, with a constant 5 V output. Show heat radiating from the regulator in proportion to Vin - Vout. AI image/animation prompt: Three pin 7805 voltage regulator with input and output capacitors, input voltage varying and output constant 5 volts, heat radiating from the package, whiteboard style, 16:9 | The 78XX series are three-terminal fixed positive voltage regulators. The last two digits give the output voltage: 7805 for 5 volts, 7812 for 12 volts, and so on. The three pins are input, common ground, and output. Inside, there is a reference, an error amplifier, a series pass transistor, and protection against overheating and short circuits. The input must be at least about 2 volts higher than the output, called the dropout. So a 7805 needs at least 7 volts at its input. Small capacitors, about 0.33 microfarad at the input and 0.1 at the output, keep it stable. The regulator behaves like a series pass transistor, so the extra voltage appears across it, and the power dissipated is the voltage difference times the load current. For high currents, a heat sink is essential. The complementary 79XX series gives negative voltages. For better efficiency at large voltage differences, switching regulators are used. But for low-noise, low-cost designs, the 78XX linear regulator is still very popular. [165 words ≈ 71 sec] |
| 4:02 – 6:07 SOLVED NUMERICAL Visual: (a) 555 astable: R1 = 1 kΩ, R2 = 10 kΩ, C = 0.1 µF. (b) 7805 with Vin = 12 V feeding a 100 Ω load. On-screen text: (a) T = 0.693 (R1 + 2R2) C = 0.693 × 21 k × 0.1 µ = 1.455 ms ; f ≈ 687 Hz (formula 1.44/((R1 + 2R2)C) = 686 Hz) tH = 0.693 × 11 k × 0.1 µ = 0.762 ms ; tL = 0.693 × 10 k × 0.1 µ = 0.693 ms Duty cycle = 11/21 = 52.4 % Capacitor swings between 1/3 and 2/3 of VCC (b) Vout = 5 V ; IL = 5/100 = 50 mA Pd = (12 − 5) × 0.05 = 0.35 W (small; a heat sink is not essential) If IL = 500 mA: Pd = 3.5 W → heat sink needed Animation: Draw the circuit with values; compute tH, tL, T, f and the duty cycle, marking them on the output waveform. For (b), show the regulator circuit and the heat dissipation bar for two load currents. AI image/animation prompt: 555 astable with values 1 kilo ohm 10 kilo ohm 0.1 microfarad and output waveform with high time 0.762 ms and low time 0.693 ms, plus a 7805 regulator power dissipation bar chart for two load currents, 16:9 | Part A. A 555 astable circuit has R 1 equal to 1 kilo-ohm, R 2 equal to 10 kilo-ohm, and C equal to 0.1 microfarad. Find the output frequency, the high and low times, and the duty cycle. Part B. A 7805 regulator is fed from 12 volts and drives a 100 ohm load. Find the load current and the power dissipated in the regulator. Part A, step one: R 1 plus 2 R 2 equals 1 plus 20, equals 21 kilo-ohm. The period T equals 0.693 into 21 kilo-ohm into 0.1 microfarad. That is 0.693 into 2.1 milliseconds, equals 1.455 milliseconds. So the frequency is about 687 hertz. The formula 1.44 over 21 kilo-ohm times 0.1 microfarad gives 686 hertz, which is the same within rounding. Step two: the high time is 0.693 into 11 kilo-ohm into 0.1 microfarad, equals 0.762 milliseconds. The low time is 0.693 into 10 kilo-ohm into 0.1 microfarad, equals 0.693 milliseconds. Together, 1.455 milliseconds. Step three: the duty cycle is 0.762 divided by 1.455, which is 52.4 percent. This agrees with R 1 plus R 2 over R 1 plus 2 R 2, which is 11 over 21. Part B. The output is 5 volts, so the load current is 5 by 100, equals 50 milliamperes. The regulator drops 12 minus 5, equals 7 volts, so it dissipates 7 into 0.05, equals 0.35 watt. This is small, and the regulator stays cool. But for a load current of 500 milliamperes, the dissipation would be 3.5 watts, and a heat sink would be essential. That is the price of a linear regulator: the extra voltage turns directly into heat. Both of these chips are cheap and simple, which is why they have been used for decades. [291 words ≈ 125 sec] |
| 6:07 – 7:05 COMMON MISTAKES AND EXAM TIPS Visual: 555 and 78XX mistakes. On-screen text: 1. The 0.693 factor is ln 2 2. Duty cycle > 50 % for the simple astable 3. Dropout: 7805 needs Vin ≥ 7 V 4. Add input and output capacitors close to the regulator Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points. One: the 0.693 factor is the natural logarithm of 2, from charging between one third and two thirds of the supply. Two: in the basic astable circuit, the duty cycle is always above 50 percent, because the capacitor charges through R 1 plus R 2 but discharges only through R 2. Three: check the dropout. A 7805 needs at least 7 volts at the input, or the output will fall below 5 volts. Four: always add the recommended capacitors close to the regulator pins, otherwise it may oscillate. And check the power dissipation: a regulator that runs too hot will shut down, thanks to its thermal protection. Real-life use: the 555 generates clocks, flashes LEDs, drives buzzers and makes simple tone generators. The 78XX family provides the stable supply for countless microcontroller boards. [135 words ≈ 58 sec] |
| 7:05 – 7:56 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. 555: three 5 kΩ → 1/3 and 2/3 VCC levels 2. Astable: tH = 0.693(R1 + R2)C ; tL = 0.693 R2 C 3. f = 1.44/[(R1 + 2R2)C] ; D = (R1 + R2)/(R1 + 2R2) 4. 78XX: fixed positive regulators; Vin ≥ Vout + 2 V 5. Pd = (Vin − Vout) × IL → heat sink Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the 555 uses a three resistor divider to set the one third and two thirds levels Two: in astable mode the high time is 0.693 R 1 plus R 2 times C, and the low time is 0.693 R 2 C Three: the frequency is 1.44 over R 1 plus 2 R 2 times C, and the duty cycle is R 1 plus R 2 over R 1 plus 2 R 2 Four: the 78XX gives a fixed positive voltage, and the input must be at least 2 volts above the output Five: the regulator dissipates V in minus V out times the load current, so a heat sink may be needed [119 words ≈ 51 sec] |
| 7:56 – 8:44 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: 555 astable: R1 = 2.2 kΩ, R2 = 22 kΩ, C = 10 nF. Find the frequency. Practice 2: A 7805 is fed from 9 V and supplies 200 mA. Find the power dissipated in the regulator. Next: Congratulations: you have completed all five units of Basic Electrical and Electronics Engineering. Revise with the practice questions in each video, and all the best for your examinations. Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A graduation cap above a stack of five unit books on a desk with a circuit board glowing behind, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a 555 astable has R 1 equal to 2.2 kilo-ohm, R 2 equal to 22 kilo-ohm and C equal to 10 nanofarad. Find the frequency. Question two: a 7805 is fed from 9 volts and supplies 200 milliamperes. Find the power dissipated in the regulator. Answers are in the pinned comment. Congratulations. You have completed all five units of Basic Electrical and Electronics Engineering. Revise with the practice questions in each video, and all the best for your examinations. If this helped, like, subscribe, and share it with your classmates. Thank you, and all the best for your exams. [113 words ≈ 48 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: f = 1.44/[(2.2k + 44k) × 10 nF] = 1.44/(4.62×10⁻⁴) = 3.12 kHz.

Practice 2 answer: Pd = (9 − 5) × 0.2 = 0.8 W.

# YouTube package

### Video title

555 Timer Astable Multivibrator & 78XX Voltage Regulator | Frequency, Duty Cycle | Solved Numerical | BEEE

### Description with chapters

Prof. Mitesh Astik explains the 555 timer in astable mode, the frequency and duty cycle equations, and the 78XX three-terminal voltage regulators, with a solved numerical.

Chapters:

0:00 Hook

0:23 Learning objectives

0:42 Inside The 555 Timer

1:31 Astable Mode: Operation

2:51 78Xx Fixed Voltage Regulators

4:02 Solved Numerical

6:07 Common Mistakes And Exam Tips

7:05 Recap

7:56 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

555 timer, astable multivibrator, 555 astable frequency duty cycle, 78XX voltage regulator, 7805 regulator, linear IC, voltage regulator IC, basic electronics, BEEE, Prof Mitesh Astik

### Thumbnail text idea

555 TIMER  |  f = 1.44/((R1 + 2R2)C)  |  7805 REGULATOR

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Mesh and nodal analysis of DC circuits Ep2. AC fundamentals: average value, RMS value, form factor and peak factor Ep3. Phasor representation of alternating quantities Ep4. Series R, L, C, RL, RC and RLC circuits Ep5. Real, reactive and apparent power and power factor Ep6. Power and energy in DC circuits and maximum power transfer |
| Unit 2 | Ep7. Single-phase transformer: principle, construction and EMF equation Ep8. Transformer losses and efficiency Ep9. DC motor: principle, back emf and torque equation Ep10. DC shunt motor characteristics and speed control Ep11. Three-phase induction motor |
| Unit 3 | Ep12. Semiconductor diode and equivalent circuits Ep13. Half-wave and full-wave bridge rectifiers Ep14. Zener diode as voltage regulator |
| Unit 4 | Ep15. BJT construction, operation and BJT as a switch Ep16. n-channel enhancement MOSFET Ep17. Common-source amplifier, DC load line and MOS inverter |
| Unit 5 | Ep18. Op-amp introduction, specifications and differential/common-mode operation Ep19. Inverting, non-inverting and summing amplifiers Ep20. Integrator, differentiator and comparator Ep21. 555 timer astable mode and 78XX voltage regulators <-- THIS VIDEO |