# Standard Test Signals & First Order System Response | Time Constant, Rise Time, Settling Time | Control Systems

Control Systems (22EEG53)  |  Unit 2: Time domain analysis  |  Presenter: Prof. Mitesh Astik

Topic: Standard Test Signals and First-Order System Response   |   Target length: 8:31 (1193 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:14 HOOK Visual: A geyser heating water; a temperature-vs-time curve that rises fast then flattens. On-screen text: Why does it heat fast, then slow? Animation: Animate a geyser tank with a thermometer; draw the exponential rise curve in sync with the thermometer. Mark one number T on the curve. AI image/animation prompt: Electric geyser with a thermometer and an exponential temperature-rise curve drawn beside it, whiteboard style, 16:9 | When you switch on a geyser, why does the water temperature rise quickly at first and then slowly settle? That curve has a name, and one number, the time constant, describes it completely. [33 words ≈ 14 sec] |
| 0:14 – 0:33 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Control Systems ¦ Unit 2: Time domain analysis By the end you can: 1. List test signals 2. Derive step response 3. Define T, tr, ts 4. Solve a numerical Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Control Systems. By the end of this video, you will be able to name the standard test signals, derive first-order step response, define time constant and settling time, and solve a numerical. Let us begin. [44 words ≈ 19 sec] |
| 0:33 – 1:46 SCENE 1: STANDARD TEST SIGNALS Visual: Four signals plotted: impulse, step, ramp, parabolic, each with its Laplace transform. On-screen text: Impulse δ(t) → 1 Step A·u(t) → A/s Ramp A·t → A/s² Parabolic A·t²/2 → A/s³ Animation: Draw the four signals in a row, each as a graph, and write its Laplace transform below. Show the pattern: each is the integral of the previous one. AI image/animation prompt: Four plots on a whiteboard: impulse, unit step, unit ramp and parabolic signal with their Laplace transforms below, 16:9 | We cannot test a system with every possible input, so we use standard test signals. The impulse is a very short, very tall pulse; its Laplace transform is 1. The step is a sudden change to a constant value, like switching on a supply; its transform is A by s. The ramp increases steadily with time, like a moving target; its transform is A by s squared. The parabolic signal is a constant acceleration; its transform is A by s cubed. Notice the pattern: each signal is the integral of the previous one. A system that follows a step well will usually follow a ramp worse and a parabola worse still, which we study later as steady-state error. Why these four? Because they represent real situations. A step is a sudden command, like setting a new temperature. A ramp is a constant-speed target, like a radar tracking an aircraft. A parabola is an accelerating target, like a missile. And an impulse is a sudden shock, like a bump on the road. [171 words ≈ 73 sec] |
| 1:46 – 2:36 SCENE 2: FIRST-ORDER SYSTEM AND ITS STEP RESPONSE Visual: RC circuit with output across the capacitor and the closed-loop form C(s)/R(s) = 1/(Ts+1). On-screen text: C(s)/R(s) = 1/(Ts + 1) Unit step: C(s) = 1/[s(Ts+1)] = 1/s − 1/(s + 1/T) c(t) = 1 − e^(−t/T) Animation: Draw the RC circuit and write its transfer function. Show partial fractions as two coloured pieces. Plot c(t) building up while a dot marks t = T at 63.2%. AI image/animation prompt: RC low-pass circuit with step source and an exponential step response curve marked at 63.2 percent at t equals T, whiteboard style, 16:9 | A first-order system has the standard form C of s by R of s equals one by T s plus one. An RC circuit, with output across the capacitor, is a perfect example, with T equal to R into C. For a unit step input, C of s equals one by s into T s plus one. Use partial fractions: one by s, minus one by s plus one by T. Take the inverse Laplace transform: c of t equals one minus e to the power minus t by T. The output starts at zero with the steepest slope, and rises towards one. There is no overshoot and no oscillation. A first-order system never overshoots a step. [117 words ≈ 50 sec] |
| 2:36 – 3:45 SCENE 3: TIME CONSTANT AND TIME-DOMAIN SPECIFICATIONS Visual: Step response curve with markers at 63.2% (t = T), 90% , 98% (t = 4T) and 10%. On-screen text: At t = T: 63.2% At t = 2T: 86.5% At t = 3T: 95% At t = 4T: 98.2% Delay time td = 0.69 T Rise time tr (10→90%) = 2.2 T Settling time ts (2%) = 4T Animation: Mark each time on the curve and drop a vertical line to the axis, writing the percentage. Show the 2% band as a shaded tube. AI image/animation prompt: Exponential step response curve with markers at 63.2 percent at T and 98 percent at 4T, shaded 2 percent tolerance band, annotated times, 16:9 | The single number T, the time constant, describes the speed. At t equals T, the output has reached 63.2 percent of its final value. At 2T, 86.5 percent. At 3T, 95 percent. At 4T, 98.2 percent, so we say the system has settled within 2 percent. Therefore, settling time for a two percent band is 4T. Other specifications: delay time is the time to reach 50 percent, which is 0.693 T. Rise time, from 10 percent to 90 percent, is T ln 9, which is 2.2 T. A smaller T means a faster system. One more result you will need in Unit 2. For a unit ramp input, the output lags the input, and the steady-state error equals T. A slower system lags more. A real-life use: a thermometer with a time constant of 10 seconds needs about 40 seconds to show the correct temperature. That is why a doctor waits for the thermometer to settle before reading it. [159 words ≈ 68 sec] |
| 3:45 – 5:27 SOLVED NUMERICAL Visual: RC circuit with R = 100 kΩ and C = 20 µF; 5 V step input; output across C. On-screen text: Given: R = 100 kΩ, C = 20 µF, step input 5 V Step 1: T = RC = 100×10³ × 20×10⁻⁶ = 2 s ; G(s) = 1/(2s + 1) Step 2: 63.2% of 5 V = 3.16 V at t = T = 2 s Step 3: td = 0.693 T = 1.386 s ; tr = 2.2 T = 4.4 s ; ts (2%) = 4T = 8 s Step 4: v(3 s) = 5(1 − e^(−1.5)) = 5 × 0.7769 = 3.884 V Step 5: unit ramp → e_ss = T = 2 units Animation: Draw the circuit and write T = RC with the numbers substituting in. Plot the 5 V response and mark each time on the axis in sequence. Compute the 3 s value with a moving cursor on the curve. Finally draw a ramp input with the lagging output and the constant gap of 2 units. AI image/animation prompt: RC circuit with 100 kilo ohm and 20 microfarad, 5 volt step response curve with markers at 2 s, 4.4 s, 8 s, and a ramp lag gap of 2 units, whiteboard style, 16:9 | A series RC circuit has R equal to 100 kilo-ohm and C equal to 20 microfarad. The output is taken across the capacitor. A 5 volt step is applied. Find the time constant, the time to reach 63.2 percent, the delay time, rise time and settling time, the output voltage at 3 seconds, and the steady-state error for a unit ramp. Step one: T equals R into C, equals 100 times 10 to the power 3, into 20 times 10 to the power minus 6, equals 2 seconds. The transfer function is one by 2 s plus 1. Step two: the final value is 5 volts, and 63.2 percent of 5 is 3.16 volts. The output reaches this at t equals T, which is 2 seconds. Step three: delay time equals 0.693 into 2, equals 1.386 seconds. Rise time equals 2.2 into 2, equals 4.4 seconds. Settling time for a 2 percent band equals 4 into 2, equals 8 seconds. Step four: the voltage at 3 seconds. V equals 5 into 1 minus e to the power minus 3 by 2. e to the power minus 1.5 is 0.2231. So V equals 5 into 0.7769, equals 3.884 volts. Step five: for a unit ramp input, the steady-state error equals the time constant, which is 2 units. The output follows the ramp, but always two units behind. Doubling the speed of the system, that is halving T, halves this error. [239 words ≈ 102 sec] |
| 5:27 – 6:58 COMMON MISTAKES AND EXAM TIPS Visual: Common first-order errors. On-screen text: 1. T is 63.2% time, not 100% time 2. ts = 4T (2%) ; 3T (5%) 3. First-order step response has NO overshoot 4. Check final value from G(0) Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four points for the exam. One: students think the output reaches its final value at t equals T. It does not; at T it reaches only 63.2 percent. Two: settling time is 4T for 2 percent and 3T for 5 percent. Always read which tolerance band the question asks for. Three: a first-order system has no overshoot. If you ever calculate an overshoot for a first-order system, something is wrong. Four: to find the final value quickly, put s equal to zero in the transfer function. For G of s equals 10 by s plus 5, G of zero is 2, so a unit step ends at 2, not 1. In the next unit, we will rely on these first-order ideas again, because every real system has one dominant time constant that sets its speed. Relate this to the s-plane. The first-order system has a single pole at s equal to minus 1 by T. The farther to the left the pole, the smaller the time constant, and the faster the system. A pole at minus 10 gives T equal to 0.1 second and a settling time of 0.4 second. Real systems span a huge range: an electronic amplifier may have a time constant in microseconds, and a heating system in minutes. [212 words ≈ 91 sec] |
| 6:58 – 7:38 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. Test signals: impulse 1, step 1/s, ramp 1/s², parabola 1/s³ 2. First order: C/R = 1/(Ts + 1) 3. Step response: c(t) = 1 − e^(−t/T) 4. 63.2% at T; ts (2%) = 4T; tr = 2.2T; td = 0.69T 5. Unit ramp steady-state error = T Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the test signals have transforms 1, one by s, one by s squared and one by s cubed Two: a first-order system has the transfer function one by T s plus one Three: its step response is one minus e to the power minus t by T, with no overshoot Four: the output reaches 63.2 percent at T, the settling time is 4T, the rise time is 2.2 T and the delay time is 0.69 T Five: for a unit ramp, the steady-state error equals T [93 words ≈ 40 sec] |
| 7:38 – 8:31 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: A first-order system has T = 0.5 s. Find its 2% settling time and the output at t = 0.5 s for a unit step. Practice 2: Put G(s) = 10/(s + 5) in standard form. Find T, the DC gain, the final value for a unit step, and ts (2%). Next: Second-Order System Response and Time-Domain Specifications Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: Four second-order step responses with different damping ratios overlaid on one graph, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: a first-order system has T equal to 0.5 seconds. Find its settling time for a 2 percent band, and the output at t equal to 0.5 seconds for a unit step. Question two: put G of s equal to 10 by s plus 5 in standard form. Find T, the DC gain, the final value for a unit step, and the 2 percent settling time. Answers are in the pinned comment. In the next video, we study the second-order system: damping ratio, natural frequency, peak overshoot, rise time and settling time. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [125 words ≈ 54 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: ts = 4T = 2 s; c(0.5) = 1 − e⁻¹ = 0.632.

Practice 2 answer: G = 2/(0.2s + 1): T = 0.2 s; DC gain = 2; final value = 2; ts = 0.8 s.

# YouTube package

### Video title

Standard Test Signals & First Order System Response | Time Constant, Rise Time, Settling Time | Control Systems

### Description with chapters

Prof. Mitesh Astik explains the standard test signals (impulse, step, ramp, parabolic) and the response of a first-order system, including time constant, rise time, delay time and settling time, with a solved RC example.

Chapters:

0:00 Hook

0:14 Learning objectives

0:33 Standard Test Signals

1:46 First-Order System And Its Step Response

2:36 Time Constant And Time-Domain Specifications

3:45 Solved Numerical

5:27 Common Mistakes And Exam Tips

6:58 Recap

7:38 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

standard test signals, first order system, time constant, step response, ramp response, settling time, rise time, control systems, 22EEG53, Prof Mitesh Astik

### Thumbnail text idea

TIME CONSTANT T  |  63.2%  |  First-Order Response in 10 min

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Transfer function of mass-spring-damper system and electrical analogy Ep2. Block diagram reduction techniques Ep3. Signal flow graphs and Mason's gain formula Ep4. Rotational mechanical systems, gear trains and torque-voltage analogy |
| Unit 2 | Ep5. Standard test signals and first-order system response <-- THIS VIDEO Ep6. Second-order system response and time-domain specifications Ep7. Type of system, static error constants and steady-state error |
| Unit 3 | Ep8. Frequency-domain specifications and time-frequency correlation Ep9. Bode plot, gain margin and phase margin |
| Unit 4 | Ep10. Stability and Routh-Hurwitz criterion Ep11. Root locus: concept and construction rules |
| Unit 5 | Ep12. Lag, lead and lag-lead compensators Ep13. P, PI and PID controllers Ep14. State-space modelling (SISO) |
| Lab videos | Lab1. Experiments 1 and 3: Second-order step response and time-domain specifications (MATLAB/Simulink) Lab2. Experiment 2: Effect of adding poles and zeros on system stability (MATLAB) Lab3. Experiment 4: Characteristics of synchro transmitter and receiver pair Lab4. Experiment 5: Root locus, breakaway point and imaginary-axis crossover (MATLAB) Lab5. Experiment 6: Frequency-domain analysis with Bode plot (MATLAB) Lab6. Experiment 7: PID controller design using tuning methods (MATLAB) Lab7. Experiment 8: DC position control system and step response (MATLAB/Simulink) Lab8. Experiments 9 and 10: Design and implementation of RC lag and lead compensators |