# MATLAB Lab: PID Controller Design & Tuning | Ziegler-Nichols and Manual Tuning | Control Systems Experiment 7

Control Systems (22EEG53)  |  Lab Experiment 7  |  Presenter: Prof. Mitesh Astik

Topic: PID Controller Design Using Tuning Methods in MATLAB   |   Target length: 8:41 (1215 narrated words at 140 wpm)   |   Audience: engineering undergraduates

## Voice and production notes

Narrator: Prof. Mitesh Astik (male, Indian English, warm and encouraging classroom tone).

Pace: about 140 words per minute. Pause ~1 second after each formula appears on screen.

TTS / voice-clone setting if not recording live: en-IN male neural voice, speed 0.95-1.0.

Say symbols aloud as written in the script (e.g. 's squared', 'zeta', 'omega n'). Say 'ohm', 'newton' naturally.

Visual style: dark-green or white whiteboard look, hand-drawn marker lines, one colour per quantity (force/voltage = red, mass/inductance = blue, damper/resistor = orange, spring/capacitor = green).

## Narrated script  (Timestamp + Scene/Visual  |  Narration)

| Timestamp + Scene / Visual | Narration (word for word) |
| --- | --- |
| 0:00 – 0:15 HOOK Visual: Three knobs labelled Kp, Ki, Kd and two step responses, bad and good. On-screen text: 62% overshoot → 13% Animation: Animate three knobs being turned; the step response redraws from a wild overshoot to a smooth settle. AI image/animation prompt: Three knobs labelled Kp Ki Kd with a step response improving from large overshoot to smooth settling, screen-capture style, 16:9 | A PID controller has three knobs. Turn them wrongly and your system overshoots by sixty percent. Turn them well, and it settles smoothly. Today we practise tuning, and we see the improvement on a graph. [35 words ≈ 15 sec] |
| 0:15 – 0:33 LEARNING OBJECTIVES Visual: Professor's name card, then a four-item checklist builds on the whiteboard. On-screen text: Prof. Mitesh Astik ¦ Control Systems ¦ Lab Experiment 7 By the end you can: 1. Build a PID 2. Apply ZN 3. Fine tune 4. Compare responses Animation: Tick marks draw themselves beside each objective as it is spoken. AI image/animation prompt: Clean whiteboard with a hand-drawn four-item checklist, friendly male Indian professor avatar in the corner, flat illustration, 16:9 | Namaste, I am Professor Mitesh Astik, and welcome to Control Systems. By the end of this video, you will be able to build a PID controller in MATLAB, apply Ziegler-Nichols settings, fine tune to meet specifications, and compare responses. Let us begin. [42 words ≈ 18 sec] |
| 0:33 – 1:05 SCENE 1: RECAP: THE PLANT AND ZN SETTINGS Visual: Plant G(s) = 1/[s(s+1)(s+5)] with Ziegler-Nichols results. On-screen text: Plant: G(s) = 1/[s(s+1)(s+5)] = 1/(s³ + 6s² + 5s) Kcr = 30 , ωcr = 2.236 rad/s , Pcr = 2.81 s ZN PID: Kp = 18 , Ti = 1.405 s , Td = 0.351 s → Ki = 12.81 , Kd = 6.32 Animation: Show the plant and the ZN table values from the PID video. AI image/animation prompt: Whiteboard recap of plant and Ziegler Nichols PID values Kp 18 Ti 1.405 Td 0.351, 16:9 | We use the plant from the PID lecture: G of s equals 1 over s into s plus 1 into s plus 5. From the Routh array, the ultimate gain is 30, the oscillation frequency is 2.236 radians per second, and the period is 2.81 seconds. The Ziegler-Nichols PID settings are Kp equal to 18, Ti equal to 1.405, and Td equal to 0.351. In parallel form, Ki is 12.81 and Kd is 6.32. [74 words ≈ 32 sec] |
| 1:05 – 1:52 SCENE 2: THE MATLAB CODE Visual: MATLAB editor with the PID script. On-screen text: G = tf(1, [1 6 5 0]); % Ziegler-Nichols Kp = 18; Ti = 1.405; Td = 0.3512; C1 = pid(Kp, Kp/Ti, Kp*Td); T1 = feedback(C1*G, 1); figure; step(T1, 40); hold on % Manual tuning C2 = pid(10, 10/3, 10*1); T2 = feedback(C2*G, 1); step(T2, 40); grid on legend('ZN','Tuned') stepinfo(T1), stepinfo(T2) % Optional: automatic tuning C3 = pidtune(G, 'PID') Animation: Highlight each code block in turn: plant, ZN controller, tuned controller. Show the two step responses overlaid, then the stepinfo results for both. AI image/animation prompt: MATLAB editor with a script building two PID controllers and an overlaid step response plot of the two closed loops, screen-capture style, 16:9 | Define the plant with tf. Create the Ziegler-Nichols controller with the pid command, which takes Kp, Ki and Kd in that order. Close the loop with feedback of C times G, with unity feedback. Plot the step response for 40 seconds. Then create a second controller, tuned manually, with Kp equal to 10, Ti equal to 3, and Td equal to 1, which in parallel form means Ki equal to 10 by 3, and Kd equal to 10. Plot it on the same axes. Use stepinfo on both closed-loop systems. As an optional extra, pidtune designs a controller automatically for the given plant. Compare what it returns with your manual settings. [111 words ≈ 48 sec] |
| 1:52 – 2:41 AIM, REQUIREMENTS AND PROCEDURE Visual: Lab sheet appears: aim, apparatus/software, numbered procedure. On-screen text: Aim: to design a PID controller using tuning methods and to study the effect of Kp, Ki and Kd on the step response. Requirements: a computer with MATLAB and the Control System Toolbox. Procedure: 1. Define the plant. 2. Find the ultimate gain and period. 3. Compute the ZN settings. 4. Simulate the ZN controller. 5. Tune manually. 6. Compare with pidtune. Animation: Each procedure step appears as a numbered card with a small screen-recording thumbnail. AI image/animation prompt: Lab sheet with aim, requirements and numbered procedure cards each with a small screenshot thumbnail, clean flat style, 16:9 | Let us write the lab sheet properly. The aim: to design a PID controller using tuning methods and to study the effect of Kp, Ki and Kd on the step response. You will need: a computer with MATLAB and the Control System Toolbox. The procedure has the following steps. Step 1: define the plant transfer function. Step 2: find the ultimate gain and the period from the Routh array or by simulation. Step 3: compute the Ziegler-Nichols PID settings. Step 4: simulate the closed loop with the ZN controller and note the specifications. Step 5: change one parameter at a time and note the effect. Step 6: compare the best manual result with pidtune. [114 words ≈ 49 sec] |
| 2:41 – 4:10 LIVE DEMO: WORKED EXAMPLE AND EXPECTED RESULTS Visual: stepinfo for the ZN controller and the manually tuned controller. On-screen text: ZN (18, 1.405, 0.3512): Overshoot ≈ 61.8 % ¦ PeakTime ≈ 1.67 s ¦ Rise (10–90%) ≈ 0.58 s ¦ Settling (2%) ≈ 10.0 s Tuned (Kp = 10, Ti = 3, Td = 1): Overshoot ≈ 12.8 % ¦ PeakTime ≈ 2.70 s ¦ Rise ≈ 0.72 s ¦ Settling ≈ 7.07 s P-only Kp = 15 (for reference): Overshoot ≈ 64 %, Settling ≈ 18 s All three have zero steady-state error for step if integral action is present (type 2 loop with PID) Expected values below were computed independently (SciPy); please confirm on your MATLAB installation. Animation: Overlay ZN, tuned, and P-only responses; annotate the overshoot of each. Then draw a three-row table of knob changes and effects: Kp ↓ (less aggressive), Ti ↑ (less integral), Td ↑ (more damping). AI image/animation prompt: Overlaid step responses for Ziegler Nichols, tuned PID and P-only with overshoot annotated, and a table of knob changes and effects, 16:9 | Run the script. The Ziegler-Nichols controller gives a fast response with a large overshoot of about 62 percent, and a long settling time of about 10 seconds. That is typical: Ziegler-Nichols tuning is aggressive, and meant as a starting point. Now the tuned controller. We reduced Kp from 18 to 10, increased the integral time from 1.4 to 3 seconds, so there is less integral action, and increased the derivative time to 1 second for extra damping. The result: overshoot falls to about 13 percent, and the settling time drops to about 7 seconds. The rise time is a little longer, 0.72 seconds instead of 0.58, which is the price of the smoother response. For reference, a P-only controller with a gain of 15 overshoots by 64 percent and takes 18 seconds to settle, so adding I and D action clearly helps. Note that the plant already has an integrator, so the steady-state error to a step is zero anyway. The integral term here improves the error to a ramp. In the lab, try changing one parameter at a time. Increase Kp: faster, but more overshoot. Increase Td: more damping. Decrease Ti: more integral action, which usually means more overshoot. Record the stepinfo values in a table. [207 words ≈ 89 sec] |
| 4:10 – 4:42 WRITING YOUR LAB RECORD Visual: Record page layout: aim, theory, code/circuit, observation table, graph, result, conclusion. On-screen text: Record layout: Aim ¦ Theory (formulas) ¦ Code/Circuit ¦ Observation table ¦ Graph ¦ Result ¦ Conclusion ¦ Viva answers Table: Kp ¦ Ti ¦ Td ¦ Mp ¦ tr ¦ ts for each trial Animation: Scroll through a model record page; each section is highlighted and a sample entry appears. AI image/animation prompt: Model lab record page with labelled sections aim theory code observation table graph result and conclusion, clean flat style, 16:9 | A good lab record has the same structure every time: aim, theory with formulas, code or circuit diagram, an observation table, graphs, the result, and a conclusion. Add a short paragraph explaining which gain you changed and why. Write the conclusion in your own words and relate it to the theory. For example, say what you observed, what the theory predicted, and why any difference exists. Marks are given for understanding, not only for output. [75 words ≈ 32 sec] |
| 4:42 – 5:33 VIVA QUESTIONS AND MODEL ANSWERS Visual: A viva board with five questions revealed one at a time, and model answers. On-screen text: Q1. What is the purpose of the integral term? Q2. What is the purpose of the derivative term? Q3. Why is the Ziegler-Nichols response so oscillatory? Q4. How do you retune the controller? Q5. Which commands did you use? Animation: Each question appears; after a pause the model answer fades in below it. AI image/animation prompt: Viva board with five question cards and model answer cards revealed one by one, clean flat style, 16:9 | Before you go to the viva, test yourself. Pause after each question and answer it aloud. Question 1. What is the purpose of the integral term? To eliminate the steady-state error. Question 2. What is the purpose of the derivative term? To add damping, reduce overshoot and improve the settling time. Question 3. Why is the Ziegler-Nichols response so oscillatory? Because the rules are designed for fast disturbance rejection, giving a quarter-amplitude decay, and the settings are aggressive for set-point changes. Question 4. How do you retune the controller? Reduce Kp, increase the integral time, and increase the derivative time, changing one parameter at a time. Question 5. Which commands did you use? tf, pid, feedback, step, stepinfo and pidtune. [120 words ≈ 51 sec] |
| 5:33 – 7:27 COMMON LAB MISTAKES AND VIVA TIPS Visual: Common lab mistakes. On-screen text: 1. Order in pid(): Kp, Ki, Kd 2. Convert Ti and Td to Ki and Kd 3. Change ONE parameter at a time 4. Always plot the same time axis Animation: Each tip appears as a red 'WRONG' note that flips to a green 'RIGHT' note with a marker tick. AI image/animation prompt: Whiteboard with red crossed-out wrong working and green corrected working side by side, exam-tips banner, flat style, 16:9 | Four tips. One: the pid command takes Kp, Ki, Kd in this order, and Ki and Kd are parallel gains, not time constants. Two: convert Ti and Td correctly: Ki is Kp by Ti, and Kd is Kp times Td. Three: when tuning, change one parameter at a time, and record the effect. If you change three together, you cannot learn which one did what. Four: use the same time axis and the same final time for all plots, so that the comparison is fair. In the viva, expect: why does ZN overshoot so much? What does the derivative term do? Why is the derivative filtered in practice? You can answer them all. Two more tips for this experiment. First, Simulink has a PID Controller block. Double-click it, and the Tune button opens an interactive tuner with sliders for response time and transient behaviour. It is a quick way to see the trade-off, but always note down the final Kp, Ki and Kd values in your record, so that you can reproduce the result. Second, save your script. A short script that sets the gains, closes the loop and prints stepinfo lets you try ten sets of gains in a few minutes, and you can paste the table straight into your observation sheet. When you compare tuning methods, define the criteria first: for example, overshoot below 15 percent and settling time below 8 seconds. Then you can say clearly which method meets the specification and which does not. In our example, the Ziegler-Nichols settings fail on overshoot, while the manually tuned controller meets both. [264 words ≈ 113 sec] |
| 7:27 – 7:50 RECAP Visual: Five bullets appear with tick marks over a faded replay of the main diagram. On-screen text: 1. pid(Kp, Ki, Kd) creates a parallel PID 2. ZN gives aggressive settings: ~62% overshoot 3. Fine tune: lower Kp, raise Ti, raise Td 4. feedback(C*G,1) closes the loop 5. pidtune gives automatic tuning Animation: Each bullet types in with a green tick; the main diagram pulses once at the end. AI image/animation prompt: Five-point recap slide on a whiteboard with green ticks and small icons, faded main diagram in the background, flat style, 16:9 | Let us recap in five points. One: the pid command takes Kp, Ki and Kd Two: Ziegler-Nichols settings are aggressive, with about 62 percent overshoot here Three: reduce Kp, increase Ti and increase Td to cut the overshoot Four: feedback of C times G closes the loop Five: pidtune provides automatic tuning as a comparison [55 words ≈ 24 sec] |
| 7:50 – 8:41 CLOSING: PRACTICE + NEXT VIDEO Visual: Two practice questions on screen; then a teaser animation for the next topic. On-screen text: Practice 1: Using the same plant, try Kp = 8, Ti = 3, Td = 0.8. Predict whether overshoot rises or falls vs the tuned case; check with stepinfo. Practice 2: Write the parallel gains Ki and Kd for Kp = 10, Ti = 3 s, Td = 1 s. Next: DC Position Control System in MATLAB/Simulink (Experiment 8) Animation: Questions slide in one by one with a 'Pause the video' icon. Teaser animation plays, then the Subscribe bell animation in the corner. AI image/animation prompt: A DC motor shaft with an encoder and a feedback loop drawn around it, with a Subscribe button and a Pause icon, 16:9 | Now it is your turn. Pause the video and try two questions. Question one: using the same plant, try Kp equal to 8, Ti equal to 3 and Td equal to 0.8. Predict whether the overshoot rises or falls compared with the tuned case, and check with stepinfo. Question two: write the parallel gains Ki and Kd for Kp equal to 10, Ti equal to 3 seconds and Td equal to 1 second. Answers are in the pinned comment. In the next video, we model a DC motor and build a position control loop in MATLAB and Simulink. If this helped, like, subscribe, and share it with your classmates. Thank you, and see you in the next lecture. [118 words ≈ 51 sec] |

## Instructor answer key (do not show on screen)

Practice 1 answer: Expected: overshoot ≈ 21 %, settling ≈ 6.8 s (less Kd relative to Kp raises overshoot).

Practice 2 answer: Ki = Kp/Ti = 3.33; Kd = Kp·Td = 10.

# YouTube package

### Video title

MATLAB Lab: PID Controller Design & Tuning | Ziegler-Nichols and Manual Tuning | Control Systems Experiment 7

### Description with chapters

Prof. Mitesh Astik demonstrates Lab Experiment 7: building a PID controller in MATLAB, applying Ziegler-Nichols settings, then fine-tuning to reduce overshoot from about 62 percent to about 13 percent.

Chapters:

0:00 Hook

0:15 Learning objectives

0:33 Recap: The Plant And Zn Settings

1:05 The Matlab Code

1:52 Aim, Requirements And Procedure

2:41 Live Demo: Worked Example And Expected Results

4:10 Writing Your Lab Record

4:42 Viva Questions And Model Answers

5:33 Common Lab Mistakes And Viva Tips

7:27 Recap

7:50 Closing: Practice + Next Video

Subscribe for the full series covering every unit of the syllabus. Practice-question answers are in the pinned comment.

### 10 tags

PID tuning MATLAB, Ziegler Nichols MATLAB, PID controller design, pid command MATLAB, pidtune, control systems lab experiment 7, step response tuning, 22EEG53 lab, Prof Mitesh Astik

### Thumbnail text idea

MATLAB LAB 6  |  PID Tuning  |  62% → 13% Overshoot

Layout: presenter cut-out on the right, big bold text on the left (yellow on dark), the key diagram (circuit / spring-damper) glowing in the centre.

# Series roadmap (syllabus coverage)

| Syllabus unit | Suggested episodes |
| --- | --- |
| Unit 1 | Ep1. Transfer function of mass-spring-damper system and electrical analogy Ep2. Block diagram reduction techniques Ep3. Signal flow graphs and Mason's gain formula Ep4. Rotational mechanical systems, gear trains and torque-voltage analogy |
| Unit 2 | Ep5. Standard test signals and first-order system response Ep6. Second-order system response and time-domain specifications Ep7. Type of system, static error constants and steady-state error |
| Unit 3 | Ep8. Frequency-domain specifications and time-frequency correlation Ep9. Bode plot, gain margin and phase margin |
| Unit 4 | Ep10. Stability and Routh-Hurwitz criterion Ep11. Root locus: concept and construction rules |
| Unit 5 | Ep12. Lag, lead and lag-lead compensators Ep13. P, PI and PID controllers Ep14. State-space modelling (SISO) |
| Lab videos | Lab1. Experiments 1 and 3: Second-order step response and time-domain specifications (MATLAB/Simulink) Lab2. Experiment 2: Effect of adding poles and zeros on system stability (MATLAB) Lab3. Experiment 4: Characteristics of synchro transmitter and receiver pair Lab4. Experiment 5: Root locus, breakaway point and imaginary-axis crossover (MATLAB) Lab5. Experiment 6: Frequency-domain analysis with Bode plot (MATLAB) Lab6. Experiment 7: PID controller design using tuning methods (MATLAB) <-- THIS VIDEO Lab7. Experiment 8: DC position control system and step response (MATLAB/Simulink) Lab8. Experiments 9 and 10: Design and implementation of RC lag and lead compensators |